✅ 2 What is this (Switch hostname & basic settings)?

This refers to the initial/basic configuration you do on every new switch, such as:

- Setting a hostname

- Configuring management access (SSH, passwords)

- Setting domain name

- Adding user accounts

- Enabling secure remote access

- Setting banner

- Saving configuration

These settings prepare the switch for secure administration and network operations.

✅ 2. Why do we do this?

We do basic switch configuration because:

To identify the device

Hostname helps in large networks where hundreds of switches exist.

To secure remote access

SSH, passwords, and usernames prevent unauthorized access.

To enable management

Basic settings allow the switch to be monitored and controlled.

To prepare for production

No switch should be deployed without minimum security and identity settings.

To standardize configurations

Organizations follow templates for consistent setups.

✅ 3. Advantages

Clear identification

Hostname makes troubleshooting and documentation easier.

Secure management

SSH + username/secret ensure safe login.

Prevent unauthorized access

Strong passwords and banners discourage intruders.

Remote administration

Network engineers can manage switches from anywhere.

Prevent configuration loss

Startup config ensures device restarts without issues.

4. Disadvantages

There are no real “disadvantages,” but a few considerations:

Misconfiguration risk

Wrong hostname or passwords may confuse teams or lock you out.

SSH requires CPU

Minimal overhead (very small), but still uses resources.

Requires proper tracking

If hostname standards are not followed → confusion in NOC.

Configuration:

Switch#

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#hostname INTHA01CS1

INTHA01CS1(config)#line console 0

INTHA01CS1(config-line)#password cisco

INTHA01CS1(config-line)#login

INTHA01CS1(config-line)#exit

INTHA01CS1(config)#enable password cisco

INTHA01CS1(config)#enable secret admin123

INTHA01CS1(config)#line vty 0 4

INTHA01CS1(config-line)#password cisco

INTHA01CS1(config-line)#login

INTHA01CS1(config-line)#exit

INTHA01CS1(config)#service password-encryption

INTHA01CS1(config)#banner motd #unauthorised user probhited#

INTHA01CS1(config)#ip default-gateway 192.168.1.1

INTHA01CS1(config)#exi

INTHA01CS1#

%SYS-5-CONFIG_I: Configured from console by console

INTHA01CS1#wr mem

Building configuration...

[OK]

INTHA01CS1#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

INTHA01CS1#

INTHA01CS1#exit

INTHA01CS1 con0 is now available

Press RETURN to get started.

unauthorised user probhited

User Access Verification

Password:

Password:

Password:

% Bad passwords

Press RETURN to get started!

unauthorised user probhited

User Access Verification

Password:

INTHA01CS1>

INTHA01CS1>en

Password:

Password:

Password:

% Bad secrets

INTHA01CS1>

INTHA01CS1>en

Password:

Password:

INTHA01CS1#