3_Assigning IP Address to a Switch for Remote Management

1. What is this?

Assigning an IP address to a switch means giving the switch a specific IP on a VLAN interface (usually VLAN 1 or a management VLAN).
This IP does not route traffic; it is used only for managing the switch through SSH, Telnet, Web GUI, or monitoring tools.

2. Why do we do this?

- To manage the switch remotely without going to the physical location.

- To access the switch via SSH/Telnet for configuration and troubleshooting.

- To monitor the switch using network tools (SNMP, SolarWinds, PRTG).

- To allow ping and basic network reachability checks.

3. Advantages

- ✔ Allows remote management (SSH/Telnet).

- ✔ Supports monitoring and alerts through NMS tools.

- ✔ Faster troubleshooting—engineers can check status from anywhere.

- ✔ Can be isolated in a secure management VLAN for better security.

4. Disadvantages

- ❌ If the IP or gateway is configured wrongly, the switch becomes unreachable.

- ❌ If not secured with ACLs/passwords, attackers may try to access it.

- ❌ Requires proper IP planning to avoid IP conflicts.

- ❌ Management VLAN failure can stop access to the switch.

Configuraition:

Switch#configure t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#interface vlan 1

Switch(config-if)#ip address 192.168.1.10 255.255.255.0

Switch(config-if)#no shutdown

Switch(config-if)#

%LINK-5-CHANGED: Interface Vlan1, changed state to up

Switch(config-if)#exit

Switch(config)#ip default-gateway 192.168.1.1

Switch(config)#line vty 0 4

Switch(config-line)#password cisco

Switch(config-line)#login

Switch(config-line)#exit

Switch(config)#exit

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#

Switch#wr mem

Building configuration...

[OK]

Switch#

Switch#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Switch#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

Switch#

Switch#

Switch#

Switch#hos

Switch#con

Switch#conf

Switch#configure t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#host

Switch(config)#hostname INPNQ01CS1

INPNQ01CS1(config)#ip domain-name .apmoller.com

INPNQ01CS1(config)#crypto key generate rsa

The name for the keys will be: INPNQ01CS1..apmoller.com

Choose the size of the key modulus in the range of 360 to 4096 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

*Mar 1 0:9:17.576: %SSH-5-ENABLED: SSH 1.99 has been enabled

INPNQ01CS1(config)#username admin secret admin123

INPNQ01CS1(config)#line vty 0 4

INPNQ01CS1(config-line)#login local

INPNQ01CS1(config-line)#transport input ssh

INPNQ01CS1(config-line)#exit

INPNQ01CS1(config)#exit

INPNQ01CS1#

%SYS-5-CONFIG_I: Configured from console by console

INPNQ01CS1#

INPNQ01CS1#wr mem

Building configuration...

[OK]

INPNQ01CS1#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

INPNQ01CS1#