4_Save Configuration and Verify Startup-Config

1. What is this?

Saving configuration means storing the running-config (current active settings) into the startup-config (the file the switch loads after reboot).
Verification means checking whether the saved configuration is correctly written to the startup memory (NVRAM).

2. Why do we do this?

- To make sure the switch keeps all changes after a reboot.

- To avoid losing important settings like VLANs, IP address, hostname, passwords, etc.

- To ensure the device boots with the correct configuration during power failure or maintenance.

3. Advantages

- ✔ Configuration remains permanent after reboot.

- ✔ Prevents loss of changes during power outages.

- ✔ Ensures stable and predictable switch behavior.

- ✔ Helps maintain consistent network setup across reboots.

4. Disadvantages

- ❌ If wrong configuration is saved, it also becomes permanent.

- ❌ Requires manual saving—if forgotten, changes are lost.

- ❌ Saving incorrect settings may cause device to become unreachable after reboot.

Configuration:

Switch>en

Switch#configure t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#hostname INPNQ01CS1

INPNQ01CS1(config)#exit

INPNQ01CS1#clock set 15:41:00 Nov 22 2025

INPNQ01CS1#sh clock

15:41:1.925 UTC Sat Nov 22 2025

INPNQ01CS1#wr mem

Building configuration...

[OK]

INPNQ01CS1#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

INPNQ01CS1#