5_Configure and Verify MAC Address Table

1. What is this?

The MAC address table (also called CAM table) is a list inside the switch that maps:
MAC Address → Switch Port → VLAN
It allows the switch to forward frames to the correct destination.
Configuring means adding static MAC entries (optional).
Verifying means checking the learned MAC addresses on each port.

2. Why do we do this?

- To see which devices are connected to which ports.

- To troubleshoot connectivity issues (like wrong VLAN or loops).

- To confirm the switch is learning MAC addresses correctly.

- To secure the network by optionally adding static MAC entries for important devices (optional).

3. Advantages

- ✔ Helps in troubleshooting (find device location quickly).

- ✔ Improves network performance by forwarding frames efficiently.

- ✔ Prevents unnecessary flooding in the network.

- ✔ Static MAC entries increase security (optional).

4. Disadvantages

- ❌ MAC table is stored in limited memory; too many devices may cause overflow.

- ❌ Static MAC entries must be updated manually (time-consuming).

- ❌ If a MAC moves to another port, it may cause confusion until the table updates.

- ❌ Wrong MAC configuration can break connectivity for devices.

Configuration:

Switch>en

Switch#sh mac address-table

Mac Address Table

-------------------------------------------

Vlan    Mac Address       Type        Ports

----    -----------       --------    -----

Switch#sh mac address-table

!after assigning the IP and pinging from one computer to other !

Switch#  sh mac address-table

Mac Address Table

-------------------------------------------

Vlan    Mac Address       Type        Ports

----    -----------       --------    -----

1    0001.64a3.26ee    DYNAMIC     Fa0/1

1    0030.f268.13e0    DYNAMIC     Fa0/2