6_Configure Duplex and Speed Settings on Switch Ports

1. What is this?

Duplex and speed settings control how a switch port communicates with connected devices.

- Speed = how fast the link operates (10/100/1000 Mbps).

- Duplex = communication mode (half-duplex or full-duplex).

Configuring this means manually setting or allowing auto-negotiation on a switch port.

2. Why do we do this?

- To ensure compatibility with older devices that don’t support auto-negotiation.

- To fix mismatches causing errors, collisions, or slow performance.

- To optimize link performance on important devices (servers, firewalls, switches).

- To stabilize links in environments with non-standard hardware.

3. Advantages

- ✔ Prevents speed/duplex mismatch issues.

- ✔ Improves link stability and performance.

- ✔ Ensures consistent and predictable port behavior.

- ✔ Useful for connecting to legacy devices (older printers, routers).

4. Disadvantages

- ❌ Wrong settings may cause collisions, CRC errors, or slow speeds.

- ❌ Manual configuration requires effort and proper documentation.

- ❌ Auto-negotiation disabled can cause mismatches with new devices.

- ❌ Troubleshooting becomes harder if inconsistent values are used across the network.

Configuration:
Switch>en

Switch#configure t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#hostname INPNQ01CS1

INPNQ01CS1(config)#interface fastEthernet 0/1

INPNQ01CS1(config-if)#speed 100

INPNQ01CS1(config-if)#duplex full

INPNQ01CS1(config-if)#exit

INPNQ01CS1(config)#interface fastEthernet 0/2

INPNQ01CS1(config-if)#speed 100

INPNQ01CS1(config-if)#duplex full

INPNQ01CS1(config-if)#exit

INPNQ01CS1(config)#exit

INPNQ01CS1#

%SYS-5-CONFIG_I: Configured from console by console

INPNQ01CS1#

INPNQ01CS1#sh interfaces fa0/1 status

Port      Name               Status       Vlan       Duplex  Speed Type

Fa0/1                        notconnect   1          a-full  a-100 10/100BaseTX

INPNQ01CS1#sh interfaces fa0/2 status

Port      Name               Status       Vlan       Duplex  Speed Type

Fa0/2                        notconnect   1          a-full  a-100 10/100BaseTX

INPNQ01CS1#wr mem

Building configuration...

[OK]

INPNQ01CS1#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

INPNQ01CS1#

INPNQ01CS1#

INPNQ01CS1#