7_Configure Description on Interfaces

1. What is this?

Configuring a description on an interface means adding a short text label to a switch port.
This description usually tells what device is connected, such as a PC, server, firewall, AP, uplink, or another switch.
Example:
description Connected to Server-1

2. Why do we do this?

- To easily identify what is connected to each port.

- To help engineers during troubleshooting or audits.

- To maintain clear documentation directly on the switch.

- To prevent mistakes when disabling or changing ports.

3. Advantages

- ✔ Makes troubleshooting faster and easier.

- ✔ No need to trace cables physically—description tells the purpose.

- ✔ Helpful during migrations, upgrades, and audits.

- ✔ Reduces errors by clearly labeling important ports.

4. Disadvantages

- ❌ Requires manual updates when devices change.

- ❌ Wrong or outdated descriptions can confuse engineers.

- ❌ If not part of standard practice, descriptions may be inconsistent.

Configuration:
Switch>en

Switch#configure t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#hostname INPNQO1CS1

INPNQO1CS1(config)#interface fastEthernet 0/1

INPNQO1CS1(config-if)#description connected to ADITYA'S PC

INPNQO1CS1(config-if)#exit

INPNQO1CS1(config)#interface fastEthernet 0/2

INPNQO1CS1(config-if)#description connected to Arya's PC

INPNQO1CS1(config-if)#exit

INPNQO1CS1(config)#exit

INPNQO1CS1#

%SYS-5-CONFIG_I: Configured from console by console

INPNQO1CS1#

INPNQO1CS1#wr mem

Building configuration...

[OK]

INPNQO1CS1#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

=============================================

INPNQO1CS1#sh running-config

Building configuration...

Current configuration : 1158 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname INPNQO1CS1

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

description connected to ADITYA'S PC

!

interface FastEthernet0/2

description connected to Arya's PC