8_Configure and Verify Switchport Modes (Access / Trunk)

1. What is this?

Switchport mode decides how a switch port handles VLAN traffic.

There are two main modes:

- Access Mode

  - Port carries traffic for only one VLAN

  - Used for PCs, printers, IP phones, servers

- Trunk Mode

  - Port carries traffic for multiple VLANs

  - Used between switch-to-switch, switch-to-router, switch-to-firewall

2. Why do we do this?

- To make sure the correct VLANs are allowed on the correct ports

- To control how devices communicate in the network

- To connect switches using trunks for inter-VLAN traffic

- To isolate user devices using access ports

- To ensure VLAN tagging works properly

3. Advantages

Access Ports

- ✔ Simple and secure (one VLAN only)

- ✔ Best for end-user devices

- ✔ Reduces VLAN misconfiguration issues

Trunk Ports

- ✔ Allows multiple VLANs over a single link

- ✔ Required for switch-to-switch communication

- ✔ Supports VLAN tagging (802.1Q)

4. Disadvantages

Access Ports

- ❌ Only one VLAN allowed (not flexible)

- ❌ Wrong VLAN assignment can break connectivity

Trunk Ports

- ❌ Incorrect allowed-VLAN settings can cause outages

- ❌ Higher risk—if trunk fails, multiple VLANs get affected

- ❌ VLAN hopping attack possible if not secured

Configuration:

enable

configure terminal

!--- Configure Access Port Fa0/1 (PC0) ---

interface fa0/1

description Connected to PC0

switchport mode access

switchport access vlan 10

exit

!--- Configure Access Port Fa0/2 (PC1) ---

interface fa0/2

description Connected to PC1

switchport mode access

switchport access vlan 10

exit

!--- Configure Trunk Port Fa0/24 (To another switch) ---

interface fa0/24

description Trunk to Switch2

switchport trunk encapsulation dot1q

switchport mode trunk

exit

!--- Exit configuration mode ---

exit

!--- Save configuration ---

write

!--- Verify ---

show interfaces fa0/1 switchport

show interfaces fa0/2 switchport

show interfaces fa0/24 switchport

show interfaces status