9_Configure VLANs and Assign Ports

1. What is this?

VLAN (Virtual LAN) is used to segregate network devices logically, even if they share the same physical switch.
Configuring VLANs means creating VLAN IDs on the switch.
Assigning ports means placing switch ports into the correct VLAN (access mode).

Example:

- VLAN 10 → Sales

- VLAN 20 → HR

- Assign gi1/0/1 to VLAN 10

2. Why do we do this?

- To segregate network traffic for security and efficiency.

- To reduce broadcast domains and improve performance.

- To organize devices logically by department or function.

- To prepare the network for inter-VLAN routing.

3. Advantages

- ✔ Better network security (departments isolated).

- ✔ Reduced broadcast traffic and network congestion.

- ✔ Easier management and troubleshooting.

- ✔ Flexibility to move devices between VLANs without changing physical cables.

4. Disadvantages

- ❌ Requires planning and documentation.

- ❌ Misconfigured VLANs can block communication.

- ❌ Adds some administrative overhead.

- ❌ Inter-VLAN routing requires a Layer-3 device.

Configuration:

enable

configure terminal

hostname INPNQ01CS1

!--- Create VLANs ---

vlan 10

name Finance

exit

vlan 20

name HR

exit

!--- Assign Ports to VLANs ---

interface fa0/1

switchport mode access

switchport access vlan 10

description Connected to Finance PC

exit

interface fa0/2

switchport mode access

switchport access vlan 20

description Connected to HR PC

exit

!--- Save Config ---

write

copy running-config startup-config

!--- Verify ---

show vlan brief

show interfaces fa0/1 switchport

show interfaces fa0/2 switchport