10A_Configure and Verify Default Gateway on Layer-2 Switch

1. What is this?

- A default gateway is the IP address of a router interface used by the switch to communicate with devices outside its local VLAN.

- Layer-2 switches forward traffic based on MAC addresses and work within the same VLAN; a default gateway allows cross-subnet communication.

- Required for remote management like SSH, Telnet, or SNMP monitoring.

- Command example:

- ip default-gateway <Gateway-IP>

e.g., ip default-gateway 192.168.1.1

2. Why do we do this?

- To enable remote management from other subnets.

- To allow monitoring and alerting from NMS tools like SolarWinds, PRTG, or Nagios.

- To send logs and notifications to central servers outside the VLAN.

- To support remote troubleshooting and updates without physical access.

- Considered a best practice for production switches.

3. Advantages

- ✔ Enables remote administration across networks.

- ✔ Supports centralized monitoring via SNMP and logging.

- ✔ Speeds up troubleshooting and issue resolution.

- ✔ Ensures network consistency for managed devices.

- ✔ Facilitates automation and scripting across subnets.

4. Disadvantages

- ❌ Misconfiguration risk: wrong gateway IP makes the switch unreachable remotely.

- ❌ Dependency on router: if the router interface is down, remote management fails.

- ❌ Not needed for local VLAN-only networks: can confuse beginners.

- ❌ Troubleshooting complexity: multiple misconfigured gateways can create issues.

- ❌ Security consideration: improper access control may expose the management VLAN to attacks.

Configuration:
On switch just add default gate way 192.168.1.1

============================================================================

Router>en

Router#configure t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#hos

Router(config)#hostname INBOM01RTR

INBOM01RTR(config)#interface GigabitEthernet 0/0

INBOM01RTR(config-if)#ip address 192.168.1.1 255.255.255.0

INBOM01RTR(config-if)#no shutdown

INBOM01RTR(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

INBOM01RTR(config-if)#exit

INBOM01RTR(config)#exit

INBOM01RTR#wr mem

Building configuration...

[OK]

INBOM01RTR#

INBOM01RTR#

INBOM01RTR#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

INBOM01RTR#

INBOM01RTR# sh ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0     192.168.1.1     YES manual administratively down down

GigabitEthernet0/1     unassigned      YES unset  administratively down down

GigabitEthernet0/2     unassigned      YES unset  administratively down down

Vlan1                  unassigned      YES unset  administratively down down

INBOM01RTR(config)#exit

INBOM01RTR#

%SYS-5-CONFIG_I: Configured from console by console

INBOM01RTR#

INBOM01RTR#