10B_Configure and verify default gateway on L2 switch

================================================

Configuration on coreswitch

enable

configure terminal

hostname INBOM25CS01

!--- Create VLANs ---

vlan 10

name Finance

exit

vlan 20

name HR

exit

!--- Access ports ---

interface fa0/1

description Finance PC

switchport mode access

switchport access vlan 10

no shutdown

exit

interface fa0/2

description HR PC

switchport mode access

switchport access vlan 20

no shutdown

exit

!--- Trunk port to router ---

interface gigabitEthernet0/1

description Link to Router

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20

no shutdown

exit

!--- Management VLAN (optional) ---

interface vlan 1

ip address 192.168.1.30 255.255.255.0

no shutdown

exit

!--- Default gateway for switch ---

ip default-gateway 192.168.1.1

exit

write memory

copy running-config startup-config

===================================================

Configuration on router

enable

configure terminal

hostname INBOM25RTR

!--- Main physical interface ---

interface gi0/0

no shutdown

!--- Subinterface for VLAN10 ---

interface gi0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

no shutdown

!--- Subinterface for VLAN20 ---

interface gi0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

exit

write memory

copy running-config startup-config