11_Create Multiple VLANs and Assign Ports with DHCP IP

1. What is this?

- VLAN (Virtual LAN) is used to logically segment a network even if devices are on the same physical switch.

- Creating multiple VLANs means assigning different VLAN IDs for departments, functions, or device groups.

- Ports are assigned to VLANs so that devices in the same VLAN can communicate directly.

- DHCP IP assigning means the devices connected to these ports receive IP addresses automatically from a DHCP server, reducing manual configuration.

- Example:

  - VLAN 10 → Sales

  - VLAN 20 → HR

  - VLAN 30 → IT

  - Ports gi1/0/1–5 → VLAN 10, etc.

2. Why do we do this?

- To segregate network traffic for performance and security.

- To reduce broadcast domains and minimize network congestion.

- To simplify management by grouping devices logically.

- To automatically assign IPs using DHCP, avoiding manual configuration errors.

- To support inter-VLAN routing if Layer-3 devices are used.

3. Advantages

- ✔ Logical separation of departments or services.

- ✔ Improved network performance by reducing broadcast traffic.

- ✔ Automatic IP assignment reduces administrative work.

- ✔ Easy management and troubleshooting.

- ✔ Flexibility to move devices between VLANs without changing physical connections.

- ✔ Supports inter-VLAN communication through routers or Layer-3 switches.

4. Disadvantages

- ❌ Requires careful planning and documentation.

- ❌ Misconfigured VLANs or DHCP scopes can cause connectivity issues.

- ❌ Additional administrative overhead for multiple VLANs.

- ❌ Inter-VLAN routing requires a Layer-3 device, adding cost.

- ❌ DHCP failure can prevent devices from obtaining IPs automatically.

Configuration:

enable

configure terminal

hostname INPNQ01L3CS

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

! Enable SSH v2

ip ssh version 2

! Create RSA keys (768 bits minimum)

crypto key generate rsa general-keys modulus 1024

ip routing

!-- VLANs

vlan 10

name Sales

exit

vlan 39

name Management

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

!-- VLAN 10

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

!-- VLAN 39

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

!-- VLAN 20

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

!-- VLAN 30

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

!-- VLAN 10

interface range Gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

!-- VLAN 20

interface range Gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

!-- VLAN 30

interface range Gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

!-- VLAN 10 - Sales

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

!-- VLAN 20 - HR

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

! Assign the PC port to VLAN 39

interface gigabitEthernet1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

! Enable SSH on VTY lines

line vty 0 4

login local

transport input ssh

exit

!-- VLAN 30 - IT

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

!-- Exclude switch IPs from DHCP

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

exit