13_Configure Trunk Ports Between Switches

1. What is this?

- A trunk port is a switch port that carries traffic for multiple VLANs between switches or between a switch and a router.

- Trunks use VLAN tagging (802.1Q) to identify which VLAN each frame belongs to.

- Configuring trunk ports ensures that VLAN information is preserved across switches.

- Example: Connecting SW1 and SW2 with a trunk port so that VLAN 10, 20, and 30 traffic can flow between them.

2. Why do we do this?

- To allow inter-switch communication for multiple VLANs.

- To extend VLANs across switches so devices in the same VLAN on different switches can communicate.

- To support inter-VLAN routing when Layer-3 routing is used.

- To ensure network consistency and scalability, especially in large networks.

- To prevent VLAN traffic isolation when multiple switches are interconnected.

3. Advantages

- ✔ Supports multiple VLANs on a single physical link.

- ✔ Reduces cabling requirements by using one trunk instead of multiple access links.

- ✔ Enables VLAN expansion across multiple switches.

- ✔ Facilitates centralized network management and monitoring.

- ✔ Preserves VLAN tagging, maintaining proper traffic segregation.

4. Disadvantages

- ❌ Misconfigured trunks can cause VLAN mismatches and network outages.

- ❌ All VLANs on the trunk are exposed to possible security risks (VLAN hopping).

- ❌ A trunk failure can affect multiple VLANs simultaneously, causing larger outages.

- ❌ Requires proper planning and documentation to avoid VLAN ID conflicts.

- ❌ Trunks may carry unnecessary VLAN traffic, slightly increasing bandwidth usage.

Configuration:

enable

configure terminal

hostname INBOM25CS1

! Domain & SSH user

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

! Enable SSH v2

ip ssh version 2

! RSA key

crypto key generate rsa general-keys modulus 1024

! Enable L3 Routing (SVI routing)

ip routing

! ------------------- VLAN CREATION -------------------

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! ------------------- SVI CONFIG -------------------

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

! ------------------- ACCESS PORTS -------------------

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

! Management PC port

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

! ------------------- TRUNK TO SW2 -------------------

interface gi1/1/4

description Trunk_to_SW2

switchport mode trunk

switchport trunk allowed vlan 10,20,30,39

no shutdown

exit

! ------------------- DHCP POOLS -------------------

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

! Excluded addresses

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

! ------------------- SSH ENABLED -------------------

line vty 0 4

login local

transport input ssh

exit

exit

wr mem

copy running-config startup-config

====================================================================

INBOM25CS2

enable

configure terminal

hostname INBOM25CS2

! Domain & SSH user

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

! Enable SSH v2

ip ssh version 2

! RSA key

crypto key generate rsa general-keys modulus 1024

! ------------------- VLAN CREATION -------------------

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! ------------------- ACCESS PORTS -------------------

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

! ------------------- TRUNK TO SW1 -------------------

interface gi1/1/4

description Trunk_to_SW1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,39

no shutdown

exit

! ------------------- SSH ENABLED -------------------

line vty 0 4

login local

transport input ssh

exit

ex

write memory

copy running-config startup-config