13_Configure Native VLAN on Trunk Ports

1. What is this?

- A native VLAN is the VLAN whose traffic is sent untagged on a trunk link.

- By default, most switches use VLAN 1 as the native VLAN.

- Configuring the native VLAN ensures untagged frames are properly identified and segregated from tagged VLAN traffic.

- Example: If VLAN 99 is set as native, all untagged traffic on the trunk will belong to VLAN 99.

2. Why do we do this?

- To avoid VLAN mismatches on trunk ports.

- To properly segregate untagged traffic from tagged VLANs.

- To enhance network security by moving the native VLAN to a non-default VLAN.

- To prevent network loops or broadcast issues caused by untagged frames.

- To maintain consistency in VLAN tagging across multiple switches.

3. Advantages

- ✔ Prevents untagged traffic confusion on trunk links.

- ✔ Enhances network security by avoiding default VLAN usage.

- ✔ Ensures trunk consistency across switches.

- ✔ Reduces misconfiguration issues in multi-switch networks.

- ✔ Supports proper VLAN segregation and traffic control.

4. Disadvantages

- ❌ Misconfigured native VLAN can lead to VLAN leaks or broadcast storms.

- ❌ All untagged traffic goes into the native VLAN, which may mix sensitive traffic if not planned properly.

- ❌ Requires documentation; otherwise, troubleshooting becomes harder.

- ❌ Native VLAN mismatch between switches can cause connectivity issues.

- ❌ Using default VLAN 1 as native may expose network to security risks.

Configuration:

enable

configure terminal

! Enable Layer 3 routing

ip routing

! ------------------- VLAN CREATION -------------------

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! ------------------- SVI INTERFACES -------------------

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

! ------------------- ACCESS PORTS -------------------

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

! Management PC port

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

! ------------------- TRUNK PORT TO SW2 -------------------

interface gi1/1/4

description Trunk_to_SW2

switchport mode trunk

switchport trunk allowed vlan 10,20,30,39

switchport trunk native vlan 39

no shutdown

exit

! ------------------- DHCP CONFIGURATION -------------------

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp excluded-address 192.168.1.10

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

ip dhcp pool MANAGEMENT

network 192.168.1.0 255.255.255.0

default-router 192.168.1.10

dns-server 8.8.8.8

exit

! ------------------- Hostname and SSH -------------------

hostname INBOM25CS1

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

line vty 0 4

login local

transport input ssh

exit

! ------------------- SAVE CONFIG -------------------

exit

write memory

copy running-config startup-config

======================================================

enable

configure terminal

! Enable Layer 3 routing

ip routing

! ------------------- VLAN CREATION -------------------

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! ------------------- SVI INTERFACES -------------------

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.2 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.2 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.2.10 255.255.255.0

no shutdown

exit

! ------------------- ACCESS PORTS -------------------

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

! Management PC port

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

! ------------------- TRUNK PORT TO SW1 -------------------

interface gi1/1/4

description Trunk_to_SW1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,39

switchport trunk native vlan 39

no shutdown

exit

! ------------------- DHCP CONFIGURATION -------------------

ip dhcp excluded-address 192.168.10.2

ip dhcp excluded-address 192.168.20.2

ip dhcp excluded-address 192.168.30.2

ip dhcp excluded-address 192.168.2.10

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.2

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.2

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.2

dns-server 8.8.8.8

exit

ip dhcp pool MANAGEMENT

network 192.168.2.0 255.255.255.0

default-router 192.168.2.10

dns-server 8.8.8.8

exit

! ------------------- Hostname and SSH -------------------

hostname INBOM25CS1

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

line vty 0 4

login local

transport input ssh

exit

! ------------------- SAVE CONFIG -------------------

exit

write memory

copy running-config startup-config