WHY ARE WE DOING ROUTER-ON-A-STICK?

Because:

A Layer-2 switch cannot route traffic between VLANs.
Each VLAN is a separate network and cannot communicate with others without a router.
Instead of buying a full Layer-3 switch, we use a single router interface for multiple VLANs — using sub-interfaces.
All VLAN traffic goes through one trunk port to the router → router routes → sends back.

This is why it's called ROUTER-ON-A-STICK (router sitting on one stick/cable).

ADVANTAGES OF ROUTER-ON-A-STICK

1. Cost-effective Inter-VLAN Routing

You don’t need a Layer-3 switch.
Only one router interface handles all VLANs.

2. Easy to configure

Just:

- Make switch port a trunk

- Create router sub-interfaces

- Add IP address + dot1Q encapsulation

Very simple for labs and small networks.

3. Full routing capability

Router supports:

- ACLs

- NAT

- QoS

- Security policies

Better than basic L3 switches.

4. Uses only ONE physical cable (one “stick”)

Instead of multiple router ports for each VLAN.

5. Perfect for Packet Tracer and CCNA labs

Simple method to teach:

- VLANs

- Trunking

- Routing

All in one setup.

DISADVANTAGE (important for interview)

Router-on-a-stick is not scalable for large enterprise networks because:

Router becomes a bottleneck
 One link handles ALL inter-VLAN traffic
 High CPU usage on router
 Only 1 Gbps throughput (if using Fast/Gig link)

This is why big networks use Layer-3 switches instead.

Configuration:

Switch 
enable

configure terminal

hostname INBOM25CS1

! --- VLANs ---

vlan 10

name Sales

vlan 20

name HR

vlan 30

name IT

vlan 39

name Management

! --- Access Ports ---

interface range Fa0/1 - 6

switchport mode access

switchport access vlan 10

interface range Fa0/7 - 12

switchport mode access

switchport access vlan 20

interface range Fa0/13 - 18

switchport mode access

switchport access vlan 30

interface Fa0/24

switchport mode access

switchport access vlan 39

! --- TRUNK to Router ---

interface gi0/2

switchport mode trunk

switchport trunk allowed vlan 10,20,30,39

ex

ex

wr mem

copy running-config startup-config

Router:

enable

configure terminal

! --- Subinterfaces for Inter-VLAN Routing ---

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface g0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface g0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

interface g0/0.39

encapsulation dot1Q 39

ip address 192.168.1.10 255.255.255.0

interface g0/0

no shutdown

! --- DHCP ---

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

end

write