Practical: Configure Inter-VLAN Routing using Layer 3 Switch

WHY ARE WE DOING INTER-VLAN ROUTING ON A LAYER 3 SWITCH?

Because:

A Layer-2 switch cannot route traffic between VLANs.

Each VLAN is a separate network and cannot communicate with others without routing.

Instead of using a separate router, a Layer-3 switch can route between VLANs directly.

Each VLAN gets an SVI (Switched Virtual Interface) with an IP address.

Traffic between VLANs is routed internally on the switch hardware, so there’s no need for an external router.

This makes inter-VLAN routing faster and more scalable than Router-on-a-Stick.

ADVANTAGES OF LAYER 3 SWITCH INTER-VLAN ROUTING

High Performance / Throughput

Routing is done in hardware (switch ASICs), so traffic is fast.

No separate router required

All VLANs are routed within the switch itself.

Saves cost and simplifies network design.

Scalable for many VLANs

Can handle dozens or hundreds of VLANs without bottleneck.

Simpler cabling

No trunk to router needed; all routing is internal.

Supports full Layer-3 features

ACLs, QoS, security policies, static and dynamic routing (OSPF, EIGRP).

Ideal for enterprise networks

Unlike Router-on-a-Stick, one switch can manage high-speed inter-VLAN traffic.

DISADVANTAGE (Important for Interview)

Layer-3 switches are more expensive than a simple router.

Configuration is slightly more complex than Router-on-a-Stick for beginners.

Not every Layer-3 switch supports advanced features (depends on model).

Configuration:

enable

configure terminal

hostname INBOM25CS1

! --- Domain & SSH ---

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

! --- Enable IP Routing ---

ip routing

! --- VLANs ---

vlan 10

name Sales

ex

vlan 20

name HR

ex

vlan 30

name IT

ex

vlan 39

name Management

ex

! --- SVI Interfaces ---

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

ex

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

ex

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

ex

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

ex

! --- Access Ports ---

interface range gi1/0/1-6

switchport mode access

switchport access vlan 10

no shutdown

ex

interface range gi1/0/7-12

switchport mode access

switchport access vlan 20

no shutdown

ex

interface range gi1/0/13-18

switchport mode access

switchport access vlan 30

no shutdown

ex

interface gi1/1/4

switchport mode access

switchport access vlan 39

no shutdown

! --- Trunk Port to Switch 2 ---

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

! --- DHCP Pools ---

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

! --- SSH on VTY ---

line vty 0 4

login local

transport input ssh

! --- Command History ---

line console 0

history size 256

line vty 0 4

history size 256