Practical: Verify VLAN Membership and Connectivity

WHY ARE WE VERIFYING VLAN MEMBERSHIP AND CONNECTIVITY?

Because:

- VLANs logically separate devices into different networks.

- A PC must be in the correct VLAN to communicate within its network.

- Inter-VLAN routing only works if devices are in the right VLANs and have correct IPs and gateways.

- Verifying ensures:

  - Each PC is assigned to its intended VLAN.

  - VLAN interfaces (SVIs) on the switch are up.

  - Inter-VLAN routing is working.

- Without verification, misconfigured ports or gateways can break communication between VLANs.

ADVANTAGES OF VERIFYING VLAN MEMBERSHIP AND CONNECTIVITY

- Ensures proper segmentation

  - Only devices in the same VLAN can communicate internally.

- Detects misconfigurations early

  - Wrong VLAN assignment, wrong trunk, or shutdown interfaces.

- Validates inter-VLAN routing

  - Confirms Layer 3 switch or router correctly routes traffic.

- Simplifies troubleshooting

  - Helps identify why a PC cannot reach another VLAN or gateway.

DISADVANTAGES / CAUTIONS

- If ignored, network issues can occur:

  - PCs cannot communicate with each other.

  - DHCP may fail if VLAN misassigned.

  - Inter-VLAN routing appears broken.

- Verification takes time in large networks if done manually.

Configuration

enable

configure terminal

hostname INBOM25CS1

! --- Domain & SSH ---

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

! --- Enable IP Routing ---

ip routing

! --- VLANs ---

vlan 10

name Sales

ex

vlan 20

name HR

ex

vlan 30

name IT

ex

vlan 39

name Management

ex

! --- SVI Interfaces ---

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

! --- Access Ports ---

interface range gi1/0/1-6

switchport mode access

switchport access vlan 10

no shutdown

interface range gi1/0/7-12

switchport mode access

switchport access vlan 20

no shutdown

interface range gi1/0/13-18

switchport mode access

switchport access vlan 30

no shutdown

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

! --- DHCP Pools ---

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

! --- Enable SSH on VTY ---

line vty 0 4

login local

transport input ssh

! --- Command History ---

line console 0

history size 256

line vty 0 4

history size 256

--- Enable Interface and VLAN Verification ---

Example commands after this config:

show vlan brief          --> Verify VLAN membership

show ip interface brief  --> Verify SVI interfaces up/up

ping 192.168.10.1       --> Test VLAN 10 connectivity

ping 192.168.20.1       --> Test VLAN 20 connectivity

ping 192.168.30.1       --> Test VLAN 30 connectivity

ping 192.168.1.10       --> Test Management VLAN