Configure Voice VLAN — Short & Sweet Theory

WHY ARE WE DOING VOICE VLAN?

Because:

- An IP Phone and a PC often connect using one single switch port.

- We must separate voice traffic from data traffic.

- Voice traffic needs higher priority so calls don’t lag.

- A dedicated Voice VLAN lets the switch apply QoS and priority tagging.

- This keeps IP Phone traffic clean and avoids interference from PC traffic.

So →
One port → Two VLANs → Data VLAN + Voice VLAN
This is why we configure Voice VLAN.

ADVANTAGES OF VOICE VLAN

1. High Voice Quality

- Prioritized packets

- Low delay, jitter, and packet loss
Perfect for IP phones.

2. Separate Broadcast Domains

- Voice devices stay isolated

- Data VLAN broadcasts don’t affect phones

3. One Port, Two Devices

- PC connects through the phone

- No need for extra cabling or ports
Efficient + Cost-saving.

4. Better Security

- Prevents PCs from joining the voice network

- Limits access to the Call Manager

5. Enterprise-Standard Setup

Used in:

- BPO, call centers

- Corporate offices

- Banks

- Any networks using IP phones

DISADVANTAGES

- Needs extra DHCP scope

- Requires correct phone model support

- Misconfigurations affect phone registration

enable

configure terminal

!creating name

hostname INBOM25AS5

! Create Voice VLAN

vlan 100

name Voice

exit

! DHCP for Voice VLAN

ip dhcp pool VOICE

network 192.168.100.0 255.255.255.0

default-router 192.168.100.1

option 150 ip 192.168.100.1

! TFTP/Call Manager

dns-server 8.8.8.8

exit

! Exclude gateway

ip dhcp excluded-address 192.168.100.1

! SVI for Voice VLAN

interface vlan 100

ip address 192.168.100.1 255.255.255.0

no shutdown

exit

! Configure Access Ports for IP Phone + PC

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

! Data VLAN for PC

switchport voice vlan 100

! Voice VLAN for IP Phone

mls qos trust cos

! Trust phone QoS marking

no shutdown

exit