20  Security — Restrict Unused Ports

WHY ARE WE DOING THIS?

Because:

- Unused switch ports are a security risk.

- Attackers can plug into these ports and access your VLAN.

- By shutting down unused ports, we prevent unauthorized access.

- This is a basic Layer 2 security best practice.

ADVANTAGES

- Prevent unauthorized access

  - Unused ports cannot be used by rogue devices.

- Reduce attacks like MAC flooding

  - Attackers can’t misuse open ports.

- Simpler network management

  - Only active ports are allowed to transmit traffic.

- Better network hygiene

  - Keeps VLANs clean and controlled.

DISADVANTAGES / LIMITATIONS

- Must manually enable ports when needed.

- If not documented, legitimate users may be blocked.

enable

configure terminal

! -------------------------

! Hostname and SSH

! -------------------------

hostname INBOM25CS1

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

! -------------------------

! Enable routing (for inter-VLAN, optional)

! -------------------------

ip routing

! -------------------------

! VLANs

! -------------------------

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! -------------------------

! SVI Interfaces (Gateway for each VLAN)

! -------------------------

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

! -------------------------

! Access Ports

! -------------------------

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

! -------------------------

! Trunk Port to Switch 2

! -------------------------

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! -------------------------

! DHCP Pools

! -------------------------

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

! -------------------------

! Enable SSH on VTY

! -------------------------

line vty 0 4

login local

transport input ssh

exit

! -------------------------

! Command History

! -------------------------

line console 0

history size 256

exit

line vty 0 4

history size 256

exit

! -------------------------

! VLAN Security — Shutdown unused ports

! -------------------------

interface range gi1/0/19 - 23

shutdown

exit

! -------------------------

! Port Security on active ports (PC-only, max 1 device)

! -------------------------

interface range gi1/0/1 - 18

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

exit

! Trunk and management ports left untouched

ex

write memory

Connecting same device after connecting same device