- Verify STP Operation (Default)

Why we do STP (Spanning Tree Protocol)

Because:

- Switches can create loops in Layer 2 networks if multiple links exist between switches.

  - Example: Two switches connected by 2 cables → a broadcast packet can circulate forever → broadcast storm.

- STP prevents loops automatically.

  - It blocks redundant paths without disabling the link physically.

- Default STP on Cisco switches (PVST+):

  - Each VLAN elects a root bridge (lowest Bridge ID).

  - Other switches calculate the shortest path to root.

  - Ports are assigned roles: Root, Designated, Alternate/Blocked.

- Redundant links can still exist.

  - If the root link fails, STP automatically unblocks a backup port → network stays up.

Advantages of STP

- Loop-free network: Prevents broadcast storms.

- Redundancy: Extra links can exist and will be used if the main path fails.

- Dynamic: Automatically adapts if switches or links are added/removed.

- VLAN-wise STP (PVST+): Each VLAN has independent root → efficient path selection.

Disadvantages / Limitations

- Convergence delay: Default STP can take ~30–50 seconds to stabilize → small downtime.

- Not ideal for large networks without enhancements like RSTP or MSTP.

- Root bridge dependent: If root fails and backup takes time → temporary loops can happen.

Configuration;

enable

configure terminal

hostname INBOM25CS1

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

! Enable SSH v2

ip ssh version 2

! RSA key

crypto key generate rsa general-keys modulus 1024

ip routing

!-- VLANs

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

!-- Access ports for PCs

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

!-- Trunks to SW2

interface gi1/1/3

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

!-- DHCP Pools

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

!-- Enable SSH

line vty 0 4

login local

transport input ssh

exit

===============================================================

enable

configure terminal

hostname INBOM25CS2

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

! Enable SSH v2

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

ip routing

!-- VLANs

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.2 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.2 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.1.11 255.255.255.0

no shutdown

exit

!-- Access ports for PCs

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

!-- Trunks to SW1

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

interface gi1/1/3

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

!-- DHCP Pools (optional, can use SW1)

ip dhcp excluded-address 192.168.10.2

ip dhcp excluded-address 192.168.20.2

ip dhcp excluded-address 192.168.30.2

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

!-- Enable SSH

line vty 0 4

login local

transport input ssh

exit