22_STP – Spanning Tree Protocol (One Page)

What is STP?

STP (Spanning Tree Protocol) is a Layer-2 protocol used in switches to prevent loops in a network.
When you connect switches with multiple paths (redundant links), loops can occur.
STP automatically blocks some ports, so the network forms a loop-free logical topology.

Why STP is Needed

Without STP, switching loops cause:

- Broadcast storms

- MAC table instability

- Duplicate frames

- Entire network outage

STP avoids this by:

- Electing a Root Bridge

- Choosing the best path to the root

- Blocking unnecessary links

So, the network stays stable even with redundancy.

Advantages of STP

1. Prevents Layer-2 loops

Stops broadcast storms and keeps network from crashing.

2. Allows redundancy

You can connect switches with multiple links safely.

3. Automatic failover

If an active link goes down, STP unblocks the backup link.

4. Stabilizes the network

Ensures predictable forwarding paths.

5. Works without manual intervention

Automatically calculates best path, roles, and port states.

Disadvantages of STP

1. Convergence is slow (traditional STP)

Takes 30–50 seconds to reconverge when a link fails.

2. Can block links unnecessarily

Some bandwidth stays unused because STP blocks redundant ports.

3. Misconfiguration can cause issues

Wrong root bridge = bad traffic flow, bottlenecks.

4. Hard to troubleshoot in large networks

Root elections, port states, and inconsistent configs can cause problems.

5. Relying only on STP is outdated

Modern networks prefer:

- RSTP (faster)

- MST (multi-instance)

- LACP/EtherChannel (loop-free bundles)

| Valid STP Priority | Explanation |
| --- | --- |
| 0 | Highest priority |
| 4096 | Next priority |
| 8192 | Next |
| 12288 | ... |
| 16384 | ... |
| 20480 | ... |
| 24576 | ... |
| 28672 | ... |
| 32768 (default) | Factory default |
| 36864 |  |
| 40960 |  |
| 45056 |  |
| 49152 | Lowest |

Configuration:

Core switch: enable

configure terminal

hostname INBOM25CS1

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

! Enable SSH v2

ip ssh version 2

! RSA key

crypto key generate rsa general-keys modulus 1024

ip routing

!-- VLANs

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

! Access ports

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

no shutdown

exit

! Management PC port

interface gi1/0/24

switchport mode access

switchport access vlan 39

no shutdown

exit

! ---- TRUNK PORT TO SWITCH 2 ----

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

exit

! ---- DHCP POOLS ----

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

! Excluded addresses

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

! Enable SSH on VTY

line vty 0 4

login local

transport input ssh

! for history

history size 256

line console 0

history size 256

line vty 0 4

history size 256

exit

! Make CS root for all VLANs

spanning-tree vlan 10,20,30,39 priority 4096

exit

=========================================

DS1

spanning-tree vlan 10,20,30,39 priority 8192

============================================

DS2

spanning-tree vlan 10,20,30,39 priority 12288

===========================================

AS

spanning-tree vlan 10,20,30,39 priority 32768

show spanning-tree vlan 10

show spanning-tree vlan 20

show spanning-tree vlan 30

show spanning-tree vlan 39

show spanning-tree summary