23. Configure STP portfast on access ports

What is STP PortFast?

PortFast is a Cisco feature for access ports (ports connected to end devices like PCs or printers).

- Normally, STP ports go through these states: Listening → Learning → Forwarding (~30–50 sec).

- PortFast skips Listening & Learning, making the port go directly to Forwarding when connected.

Why We Use PortFast

- End devices need fast connectivity: PCs, servers, and VoIP phones should start communicating immediately.

- Prevents unnecessary delays: Without PortFast, DHCP requests or login scripts may fail during STP delay.

- Safe for edge/access ports only: Not for uplinks or trunks (prevents loops).

Advantages of PortFast

- Immediate network access → devices can send/receive traffic as soon as they connect.

- Faster DHCP and bootup → avoids timeout issues due to STP delay.

- Improves user experience → no network wait time for PCs or VoIP phones.

- Simple to configure → per port or globally.

Disadvantages / Risks

- Can create loops if misused → do not enable on trunk ports or ports connecting to other switches.

- False sense of security → network admins may forget it’s only safe on edge ports.

- Does not replace STP → only changes port startup behavior, loops still prevented by STP elsewhere.

enable

configure terminal

hostname INBOM25CS1

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

! Enable SSH

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

ip routing

! --- GLOBAL STP SETTINGS ---

spanning-tree mode rapid-pvst

spanning-tree portfast default

! --- VLANs ---

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! --- SVIs ---

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

interface vlan 39

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

! --- ACCESS PORTS ---

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

spanning-tree portfast

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

spanning-tree portfast

no shutdown

exit

interface gi1/0/24

switchport mode access

switchport access vlan 39

spanning-tree portfast

no shutdown

exit

! --- TRUNK LINKS ---

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

interface gi1/1/3

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! --- DHCP POOLS ---

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

ip dhcp pool IT

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

exit

! --- EXCLUDED ADDRESSES ---

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

! --- SSH VTY ---

line vty 0 4

login local

transport input ssh

exit

! --- HISTORY ---

line console 0

history size 256

exit

line vty 0 4

history size 256

exit

=========================================

enable

conf t

hostname INBOM25DS1

spanning-tree mode rapid-pvst

spanning-tree portfast default

! VLANs

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! Uplink to Core

interface gi1/1/3

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Downlink to DS2 or Access Switch

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Example Access Ports

interface range gi1/0/1 - 12

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

interface range gi1/0/13 - 24

switchport mode access

switchport access vlan 20

spanning-tree portfast

no shutdown

exit

=======================================================

enable

conf t

hostname INBOM25DS2

spanning-tree mode rapid-pvst

spanning-tree portfast default

! VLANs

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! Uplink to Core

interface gi1/1/3

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Link to DS1 or AS

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Example Access Ports

interface range gi1/0/1 - 12

switchport mode access

switchport access vlan 30

spanning-tree portfast

no shutdown

exit

interface range gi1/0/13 - 24

switchport mode access

switchport access vlan 20

spanning-tree portfast

no shutdown

exit

===============================

enable

configure terminal

hostname ASW1

! ------------------------------------------------------

! BASIC SETTINGS

! ------------------------------------------------------

no ip domain-lookup

ip domain-name adityalab.com

username admin privilege 15 secret qwertyuiop

ip ssh version 2

crypto key generate rsa general-keys modulus 1024

! ------------------------------------------------------

! VLANs (must match DS1/DS2)

! ------------------------------------------------------

vlan 10

name Sales

exit

vlan 20

name HR

exit

vlan 30

name IT

exit

vlan 39

name Management

exit

! ------------------------------------------------------

! ACCESS PORTS FOR PCs

! ------------------------------------------------------

interface range gi1/0/1 - 6

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

interface range gi1/0/7 - 12

switchport mode access

switchport access vlan 20

spanning-tree portfast

no shutdown

exit

interface range gi1/0/13 - 18

switchport mode access

switchport access vlan 30

spanning-tree portfast

no shutdown

exit

! ------------------------------------------------------

! MANAGEMENT VLAN INTERFACE

! (Only if needed — otherwise remove)

! ------------------------------------------------------

interface vlan 39

ip address 192.168.1.11 255.255.255.0

no shutdown

exit

! ------------------------------------------------------

! TRUNK LINKS TO DS1 & DS2

! ------------------------------------------------------

! TRUNK TO DS1

interface gi1/1/3

switchport mode trunk

switchport trunk allowed vlan all

spanning-tree portfast trunk

no shutdown

exit

! TRUNK TO DS2

interface gi1/1/4

switchport mode trunk

switchport trunk allowed vlan all

spanning-tree portfast trunk

no shutdown

exit

! ------------------------------------------------------

! ENABLE RSTP

! ------------------------------------------------------

spanning-tree mode rapid-pvst

! ------------------------------------------------------

! ENABLE HISTORY

! ------------------------------------------------------

line console 0

history size 256

line vty 0 4

login local

transport input ssh

history size 256

exit