24_Configure BPDU Guard

1. What is BPDU Guard?

BPDU Guard is a Spanning-Tree protection feature that shuts down a port if it receives a BPDU (Bridge Protocol Data Unit).

In simple words:

- Port fast ports should connect only to end devices (PC, printer, server).

- If a switch is accidentally connected → it sends BPDU →
BPDU Guard immediately disables the port (err-disable).

2. Why do we use BPDU Guard?

To prevent accidental or malicious switches from joining the network and creating STP loops.

REAL REASONS:

✔ Prevents loops
✔ Protects Root Bridge
✔ Stops unauthorized switches
✔ Ensures end-user ports stay safe

3. Advantages

Best protection for access ports

If someone plugs a small unmanaged switch into a port, BPDU Guard shuts the port instantly.

Protects against STP manipulation

Stops someone from plugging a switch with lower STP priority and becoming Root Bridge.

Works with PortFast

PortFast + BPDU Guard = Safe user ports

4. Disadvantages

Port goes down immediately if misconfigured

If you accidentally enable BPDU Guard on a trunk, the port will shut down.

Users may lose access

If a user connects a cheap router/switch → port shuts down → helpdesk ticket.

Requires monitoring

Ports in err-disable need manual recovery (or auto recovery timer)