25_Configure BPDU Filter

1️⃣ What is BPDU Filter?

- BPDU = Bridge Protocol Data Unit

- BPDU Filter is a feature on Cisco switches that prevents sending or receiving BPDUs on a port.

- Used to disable spanning-tree participation for certain ports.

👉 Simple words:
BPDU Filter = Makes a port “invisible” to STP

2️⃣ Why Do We Use BPDU Filter?

- Edge ports / Access ports

  - Ports connected to PCs or servers don’t need to participate in STP

- Prevent accidental topology changes

  - If a user plugs in another switch, it could create loops

- Enhance performance

  - Edge ports don’t process BPDUs unnecessarily

3️⃣ BPDU Filter Behavior

| Scenario | Behavior |
| --- | --- |
| BPDU received on port | Port shuts BPDU filtering (if configured) |
| BPDU not received | Port continues as normal access port |
| Enabled globally | All edge ports filtered until BPDU received |

4️⃣ BPDU Filter vs BPDU Guard

| Feature | BPDU Filter | BPDU Guard |
| --- | --- | --- |
| Prevents sending/receiving BPDUs | ✅ | ❌ |
| Shuts down port on BPDU reception | ❌ | ✅ |
| Used on | Edge ports | Edge ports |
| Goal | STP transparency | STP safety |

5️⃣ Advantages

- Prevents unnecessary STP processing

- Protects edge devices from loops

- Reduces CPU usage

6️⃣ Disadvantages

- If misconfigured → can create loops

- Disables STP protection on port

7️⃣ Real-World Use Case

- Switch port connected to a PC or printer

- Enable BPDU filter → port participates in STP only if a switch is plugged in

8️⃣ One-Line Purpose (Interview Ready)

BPDU Filter disables STP BPDUs on edge ports to prevent unnecessary spanning-tree participation and accidental topology changes.

9️⃣ Devices Required

- 1 Switch

- 1 PC (Edge port)

Configuration:

BPDU Filter – Full Configuration

Scenario

- Fa0/2 → Edge port connected to a PC

- Enable BPDU Filter on this port

STEP 1️⃣ – Enable BPDU Filter Globally (Optional)

enable

configure terminal

spanning-tree portfast bpdufilter default

end

Explanation:

- All PortFast ports automatically filter BPDUs

- Used for edge ports globally

STEP 2️⃣ – Enable PortFast on Edge Port

enable

configure terminal

interface fa0/2

spanning-tree portfast

end

Explanation:

- Marks port as edge port (connected to end device)

- Required for BPDU filter to work

STEP 3️⃣ – Enable BPDU Filter on Specific Port

enable

configure terminal

interface fa0/2

spanning-tree bpdufilter enable

end

Explanation:

- Filters BPDUs only on this port

- Port does not send or receive BPDUs

STEP 4️⃣ – Verify BPDU Filter

show spanning-tree interface fa0/2 detail

show spanning-tree summary

Look for:

- PortFast enabled

- BPDU filter status: enabled

STEP 5️⃣ – Disable BPDU Filter

enable

configure terminal

interface fa0/2

no spanning-tree bpdufilter enable

end

INTERVIEW / EXAM READY LINE

BPDU Filter is used on edge ports to prevent them from participating in STP, avoiding unnecessary BPDU traffic and potential topology changes.