What is Root Guard?

- Root Guard prevents unauthorized switches from becoming Root Bridge.

- Applied on designated ports (towards access switches).

👉 Simple words:
Root Guard locks the Root Bridge position

Why Do We Use Root Guard?

- Root Bridge should be core / distribution switch

- If someone plugs a cheap switch with lower priority → it may become root

- That can break:

  - Traffic paths

  - Network performance

👉 Root Guard stops this

What Happens When Root Guard Triggers?

- If a port receives superior BPDU:

  - Port goes to root-inconsistent state

  - Port is blocked

- When superior BPDU stops:

  - Port automatically recovers

Root Guard vs BPDU Guard

| Feature | Root Guard | BPDU Guard |
| --- | --- | --- |
| Prevents new root | ✅ | ❌ |
| Shuts down port | ❌ | ✅ |
| Port state | Root-inconsistent | Err-disabled |
| Used on | Switch-to-switch | Edge ports |

Configuration:

enable

configure terminal

spanning-tree vlan 1 priority 4096

interface GigabitEthernet 1/0/1

spanning-tree guard root

ex

TRY TO MAKE 2nd switch as root

enable

configure terminal

spanning-tree vlan 1 priority 0

end

EXPECTED RESULT

Port on SW1 goes to root-inconsistent

SW2 cannot become root

Root remains SW1

VERIFY

show spanning-tree inconsistentports

show spanning-tree vlan 1

INTERVIEW ONE-LINE

Root Guard prevents downstream switches from becoming the STP root bridge.