1️⃣ What is Loop Guard?

- Loop Guard is a Spanning Tree Protocol (STP) protection feature.

- It prevents Layer-2 loops caused by unidirectional link failures.

- When enabled, Loop Guard ensures that STP blocked ports do not accidentally move to forwarding state.

👉 Simple words:
Loop Guard stops blocked ports from becoming active when BPDUs stop arriving

2️⃣ Why Do We Need Loop Guard?

In STP:

- Some ports are blocked to prevent loops.

- These blocked ports expect to receive BPDUs.

Problem:

- If BPDUs stop arriving due to:

  - Unidirectional link

  - Fiber issue

  - Software bug

- The switch may assume the link is safe and move port to forwarding, causing a loop.

👉 Loop Guard prevents this situation

3️⃣ What Problem Does Loop Guard Solve?

| Without Loop Guard | With Loop Guard |
| --- | --- |
| Blocked port may forward | Blocked port stays blocked |
| High chance of loop | Loops prevented |
| Broadcast storms | Stable network |

4️⃣ How Loop Guard Works

- Port is in blocking state

- Switch expects periodic BPDUs

- BPDUs stop arriving

- Loop Guard puts port into loop-inconsistent state

- Port does NOT forward traffic

- When BPDUs return → port recovers automatically

5️⃣ Where Do We Use Loop Guard?

- Switch-to-switch links

- Redundant links

- Distribution & Core layer

- Fiber links prone to unidirectional failure

6️⃣ Loop Guard vs Root Guard

| Feature | Loop Guard | Root Guard |
| --- | --- | --- |
| Protects against | BPDU loss | Superior BPDU |
| Port state | Loop-inconsistent | Root-inconsistent |
| Use case | Redundant links | Access/downstream links |

7️⃣ Advantages of Loop Guard

- Prevents Layer-2 loops

- Protects STP topology

- Automatic recovery

- No manual intervention needed

8️⃣ Disadvantages of Loop Guard

- Limited Packet Tracer behavior

- Misconfiguration may block valid links

- Requires STP to be enabled

9️⃣ Real-World Example

- Two switches connected via fiber

- Fiber RX fails → no BPDUs received

- Without Loop Guard → port forwards → loop

- With Loop Guard → port blocks safely

🔟 One-Line Purpose (Interview Ready)

Loop Guard prevents STP blocked ports from transitioning to forwarding state when BPDUs are unexpectedly lost.

⚠️ Important Notes

- Loop Guard is applied on non-edge ports

- Not used on PortFast ports

- Works best with Root Guard in hierarchical designs

Loop Guard don’t work on the Cisco Packet tracer

Configuration:

enable

configure terminal

! Enable STP

spanning-tree mode pvst

! (Optional) Make this switch Root Bridge

spanning-tree vlan 1 priority 4096

! Enable Loop Guard globally (recommended)

spanning-tree loopguard default

! OR enable Loop Guard on specific interface

interface fa0/1

spanning-tree guard loop

exit

end

Check:

show spanning-tree summary

show spanning-tree inconsistentports

Removal:

enable

configure terminal

no spanning-tree loopguard default

interface fa0/1

no spanning-tree guard loop

end

ONE-LINE INTERVIEW ANSWER

Loop Guard prevents STP blocked ports from becoming forwarding when BPDUs are lost due to unidirectional link failures.