28_Verify STP topology changes
What is an STP Topology Change?

- A topology change happens when:

  - A port goes up or down

  - A switch is added or removed

  - A blocked port becomes forwarding

- STP recalculates the tree to avoid loops

👉 Simple words:
Any physical or logical change in switch links = topology change

Why Do We Verify STP Topology Changes?

- To check network stability

- To detect flapping links

- To identify misconfigured ports

- To troubleshoot broadcast storms & delays

What Happens During Topology Change?

- Switch sends TCN (Topology Change Notification)

- CAM table ages faster

- Temporary traffic disruption may occur

What Can Cause Frequent Topology Changes?

- PC ports without PortFast

- Unstable cables

- Bad NICs

- Incorrect STP configuration

VERIFY STP TOPOLOGY CHANGES

ALL COMMANDS –

enable

! Show STP status for VLAN

show spanning-tree vlan 1

! Show number of topology changes and last change time

show spanning-tree detail

! Show root bridge information

show spanning-tree root

! Show STP summary

show spanning-tree summary

! Show ports causing topology changes

show spanning-tree interface fa0/1 detail

! Show STP events (if supported)

show spanning-tree detail | include change

WHAT TO LOOK FOR

- Number of topology changes

- Time since last change

- Which interface caused it

- Root bridge stability

HOW TO REDUCE TOPOLOGY CHANGES

interface range fa0/2 - 24

spanning-tree portfast

INTERVIEW ONE-LINE

STP topology changes indicate link or port state changes and are verified to ensure network stability and prevent loops.