29_Configure Rapid PVST+

1️⃣ What is Rapid PVST+?

- Rapid PVST+ is a Cisco STP mode based on IEEE 802.1w (RSTP).

- It runs one Rapid Spanning Tree instance per VLAN.

- Provides very fast convergence compared to classic STP (802.1D).

👉 Simple words:
Rapid PVST+ = Faster STP + per-VLAN control

2️⃣ Why Do We Use Rapid PVST+?

Classic STP problems:

- Slow convergence (30–50 seconds)

- Network outage during link failure

Rapid PVST+:

- Recovers in 1–3 seconds

- Minimizes downtime

- Improves network stability

👉 Used in modern enterprise networks

3️⃣ What Problem Does It Solve?

| Classic STP | Rapid PVST+ |
| --- | --- |
| Slow recovery | Fast recovery |
| Long outage | Minimal outage |
| Inefficient | Optimized |

4️⃣ How Rapid PVST+ Works

- Uses proposal/agreement mechanism

- Ports have new roles:

  - Root

  - Designated

  - Alternate

  - Backup

- Edge ports move immediately to forwarding

5️⃣ Port States Comparison

| STP (802.1D) | Rapid PVST+ |
| --- | --- |
| Blocking | Discarding |
| Listening | Discarding |
| Learning | Learning |
| Forwarding | Forwarding |

6️⃣ Rapid PVST+ Port Types

- Edge Port → Like PortFast

- Point-to-Point → Full-duplex links

- Shared → Half-duplex links

7️⃣ Advantages of Rapid PVST+

- Very fast convergence

- Per-VLAN load balancing

- Backward compatible with STP

- Reduces downtime

8️⃣ Disadvantages of Rapid PVST+

- Cisco proprietary

- More CPU usage than STP

- Limited support on non-Cisco devices

9️⃣ Real-World Use Case

- Core & Access switches

- Redundant uplinks

- Voice & video networks

🔟 One-Line Purpose (Interview Ready)

Rapid PVST+ provides fast per-VLAN spanning tree convergence to prevent loops and minimize network downtime.

anning-tree root