What is Static EtherChannel?

- EtherChannel = bundling multiple physical switch links into one logical link.

- Static EtherChannel means the bundle is created manually without negotiation protocols (no LACP, no PAgP).

- All member ports act as one single logical interface (Port-Channel).

- Switch treats 2, 4, or 8 links as one big pipe → combined bandwidth.

- If one link fails, traffic continues on remaining links → no downtime.

- Both sides must be configured exact same way (same mode, speed, VLAN, trunk/access).

- Port-Channel interface appears in configuration as interface Port-channel X.

- Load balancing happens at Layer 2 or Layer 3: per destination MAC/IP etc.

- Reduces spanning tree complexity because STP sees one logical link.

- Static EtherChannel uses:

- channel-group X mode on

- No dynamic negotiation.

2️⃣ Why do we use Static EtherChannel? (Purpose)

- To increase bandwidth between switches, servers, routers.

- To provide redundancy (if 1 link fails, no outage).

- To avoid STP blocking ports (STP sees 1 logical link only).

- To combine multiple cheap Gigabit links instead of buying 10G ports.

- To stabilize inter-switch communication in enterprise networks.

- To improve performance for heavy traffic, VLAN trunking, VTP, DTP.

- To reduce congestion on uplink links.

- To create reliable uplinks for distribution ↔ core switches.

- To support link aggregation for server NIC teaming.

- To allow consistent bandwidth for VLAN trunk carrying many VLANs.

3️⃣ Advantages of Static EtherChannel

- Simple and predictable (No negotiation protocol involved).

- Fast convergence — no LACP/PAgP handshake delays.

- Provides extra bandwidth by combining multiple links.

- Offers high availability — failure of one link doesn’t bring down port-channel.

- Load balancing across all active links.

- Compatible with many devices (even those without LACP/PAgP support).

- Easy for labs, Packet Tracer, and basic network setups.

- STP treats it as one link → fewer blocked ports.

- Works well for servers using NIC bonding.

- No protocol overhead → slightly better performance.

4️⃣ Disadvantages of Static EtherChannel

Point-wise:

- No automatic negotiation → higher chance of misconfiguration.

- If settings mismatch on both sides → port-channel stays down.

- Doesn’t detect wrong cables or wrong port connections.

- Not scalable for large networks (LACP is preferred).

- Harder troubleshooting due to manual configurations.

- Adding and removing links must be done with care — otherwise flaps.

- Cannot detect unidirectional links.

- No automatic protection against loops.

- Not recommended for production → LACP is safer.

- Requires strict identical configuration on all member ports.

✅ Maximum Number of Links in EtherChannel

Cisco supports:

- Maximum 8 links active in a single EtherChannel (Port-Channel).

- Some platforms also support 8 active + 8 standby (LACP).

So maximum active links = 8.

✅ Maximum Link Speeds Supported

Depending on platform:

- FastEthernet (100 Mbps)

- GigabitEthernet (1 Gbps)

- 10-GigabitEthernet (10 Gbps)

- 25-Gig (25 Gbps) (newer switches)

- 40-Gig (40 Gbps)

- 100-Gig (100 Gbps)

Configuration:
SW1

En

Config t

interface range gi0/1 - 8

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode on

exit

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

exit

En

Config t

interface range gi0/1 - 9

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode on

exit

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

exit

verification

show etherchannel summary

show interfaces trunk

show running-config interface port-channel 1