32_Configure EtherChannel using PAgP

1️⃣ What is PAgP?

- PAgP = Port Aggregation Protocol
Cisco-proprietary protocol used to automatically create EtherChannel.

- Helps multiple physical links behave like one single logical link (Port-Channel).

- Negotiates and checks:

  - Speed

  - Duplex

  - VLAN info

  - Trunk/access mode

- Ensures both sides agree before forming the channel.

2️⃣ Why do we use PAgP? (Purpose)

- To increase bandwidth by combining multiple interfaces.

- To provide redundancy (if one link fails, traffic continues on others).

- To allow load balancing across multiple links.

- To reduce Spanning-Tree blocking (STP treats EtherChannel as one link).

- To remove manual configuration errors by using automatic negotiation.

3️⃣ How PAgP Works (Modes)

| Mode | Meaning | Channel Forms With |
| --- | --- | --- |
| auto | Passive — waits for negotiation | desirable |
| desirable | Actively tries to form EtherChannel | auto or desirable |

⚠️ auto + auto = No EtherChannel
⚠️ Works only on Cisco devices

4️⃣ Advantages of PAgP

- Auto-negotiation avoids mismatch errors.

- Cisco-proprietary — optimized for Cisco switches.

- Detects mismatched configs and prevents unstable channels.

- Saves troubleshooting time by validating link parameters.

- Supports load balancing, fault tolerance, higher throughput.

5️⃣ Disadvantages of PAgP

- Cisco-only — cannot be used with other vendors (HP, Juniper, Arista).

- Slower than static EtherChannel because negotiation takes time.

- Limited compared to LACP (IEEE 802.3ad) which is more flexible.

- auto + auto mode confusion (many beginners misconfigure).

- If misconfigured VLAN or speed, PAgP silently fails to form channel.

6️⃣ When to Use PAgP?

✔ When both switches are Cisco.
✔ When you want automatic negotiation.
✔ When you want safer channel creation with validation.

Configuration:

The below configuration is for 2960, 2950, 3560 use ONLY dot1q

enable

configure terminal

hostname INBOMCS01

! Use the interfaces you are bundling

interface range gig1/0/1 - 2

switchport mode trunk

switchport trunk encapsulation dot1q

switchport trunk allowed vlan all

channel-group 1 mode desirable

no shutdown

exit

! Create the Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk encapsulation dot1q

switchport trunk allowed vlan all

no shutdown

exit

-----------------------------------------------

enable

configure terminal

hostname INBOMAS01

! Use the interfaces you are bundling

interface range gig1/0/1 - 2

switchport mode trunk

switchport trunk encapsulation dot1q

switchport trunk allowed vlan all

channel-group 1 mode auto

no shutdown

exit

! Create the Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk encapsulation dot1q

switchport trunk allowed vlan all

no shutdown

exit

=======================================================

FOR L3 3650

enable

configure terminal

interface range gi1/0/1 - 2

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode desirable

no shutdown

exit

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

--------------------------------------------------

enable

configure terminal

interface range gi1/0/1 - 2

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode auto

no shutdown

exit

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

Checking

show etherchannel summary

show interfaces port-channel 1

show spanning-tree interface port-channel 1

========================================================

SW1(config)# interface gi1/0/3

SW1(config-if)# switchport mode access

SW1(config-if)# switchport access vlan 10

SW1(config-if)# no shutdown

SW1(config-if)# exit

SW2(config)# interface gi1/0/3

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 10

SW2(config-if)# no shutdown

SW2(config-if)# exit

PC connections

IP: 10.0.10.1

Subnet: 255.255.255.0

IP: 10.0.10.2

Subnet: 255.255.255.0