33. What is LACP EtherChannel?

- EtherChannel is a technology that allows multiple physical links between switches or devices to be combined into a single logical link.

- LACP (Link Aggregation Control Protocol) is the IEEE 802.3ad standard protocol used for forming and managing EtherChannels dynamically.

- LACP negotiates between switches to ensure that all physical links in the bundle are compatible and active.

- Each physical interface in the EtherChannel is treated as a member port. The logical interface, called a Port-Channel, carries all traffic across the member ports.

- LACP uses two modes:

  - Active → Initiates negotiation and sends LACP packets

  - Passive → Responds to negotiation only

- EtherChannel can be configured for Layer 2 (switch-to-switch trunking) or Layer 3 (routed port-channel).

2. Why are we doing this? (Purpose of LACP EtherChannel)

- Increase bandwidth – Multiple 1 Gbps links can be combined to act as a single higher-bandwidth link (e.g., 2 x 1 Gbps = 2 Gbps logical link).

- Provide redundancy – If one physical link fails, traffic continues on remaining links without affecting network connectivity.

- Reduce STP blocking – In normal STP (Spanning Tree Protocol), redundant links are blocked to prevent loops. EtherChannel bundles multiple links into one logical link, reducing blocked ports.

- Dynamic negotiation – LACP automatically ensures compatibility between devices and prevents misconfigurations.

- Load balancing – Traffic can be distributed across all active links based on a hash of source/destination IP or MAC addresses.

- Simplified management – Multiple physical links can be managed as one Port-Channel interface, reducing configuration complexity.

3. Advantages of LACP EtherChannel

- Increased Bandwidth

  - Aggregates multiple physical links into one logical link, giving higher throughput.

- High Availability / Redundancy

  - If one link fails, the other links continue to carry traffic, reducing downtime.

- STP Efficiency

  - Spanning Tree sees EtherChannel as a single link, avoiding blocked redundant links.

- Dynamic Negotiation and Error Prevention

  - LACP ensures all member links are compatible before bundling, preventing misconfiguration.

- Load Balancing

  - Distributes traffic evenly across links using source/destination IP or MAC addresses.

- Simplified Configuration

  - Configure multiple links as one logical interface (Port-Channel) instead of configuring each link individually.

- Standards-Based

  - Unlike PAgP (Cisco proprietary), LACP works with multi-vendor devices.

- Flexible Modes

  - Active and Passive modes allow controlled negotiation of links.

4. Disadvantages of LACP EtherChannel

- Limited by Physical Interface Count

  - Maximum number of physical links per EtherChannel varies (usually 8).

- Traffic Hashing Limitation

  - Load balancing is based on a hash algorithm. If many flows use the same hash, some links may be underutilized.

- Complex Troubleshooting

  - Misconfiguration on one member link can cause the entire EtherChannel to fail.

- Hardware/Platform Limitations

  - Some older switches may not support LACP or have fewer supported member links.

- STP Misunderstanding

  - If STP is not properly configured on Port-Channels, loops can still occur in the network.

- Requires Consistent Configuration

  - Speed, duplex, VLANs, and trunk settings must match on all member interfaces.

Configuration:

enable

configure terminal

hostname INBOMCS01

interface range gi1/0/1 - 2

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode active

no shutdown

exit

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

===========================================

enable

configure terminal

hostname INBOMDS01

interface range gi1/0/1 - 2

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode passive

no shutdown

exit

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

checking

show spanning-tree interface port-channel 1

show etherchannel summary

show lacp neighbor

show interfaces port-channel 1