35 EtherChannel Load Balancing – Overview

1️⃣ What is this?

- Load balancing in EtherChannel decides how traffic is distributed across member links.

- By default, traffic is sent on one link for a specific flow (source/destination) to avoid packet reordering.

- Cisco switches use hash algorithms to decide which physical link carries a frame.

- Hash options include:

  - Source MAC

  - Destination MAC

  - Source + Destination MAC

  - Source IP

  - Destination IP

  - Source + Destination IP

Example: Frames from PC1 → PC2 always use the same link, but traffic from PC3 → PC4 can use another link.

2️⃣ Why we do this

- To utilize all member links in the EtherChannel.

- To increase throughput and avoid congestion on a single link.

- To balance traffic dynamically across multiple links.

- To maintain packet order for a single conversation (important for TCP).

3️⃣ Advantages

- Better bandwidth utilization – All links can carry traffic.

- Redundancy – If one link fails, other links continue.

- TCP session safety – Hash ensures packets from same session stay on same link.

- Flexible algorithms – Can choose MAC or IP-based hashing depending on network requirements.

4️⃣ Disadvantages

- Uneven traffic distribution – Hash may not split traffic equally if flows are few.

- Limited to per-flow load balancing – Does not split a single TCP session over multiple links.

- Some older devices do not support all hash methods.

- Misconfiguration can lead to one link being over-utilized.

Configuration:

Pending