37 What is EtherChannel with Access Ports?

EtherChannel can also be used with access ports (ports that carry a single VLAN).
Multiple physical access ports → 1 logical Port-Channel.

Example:
Gi1/0/1 and Gi1/0/2 assigned to VLAN 10 → bundled → logical Port-Channel 1 (still VLAN 10).

✅ Why do we use EtherChannel for Access Ports?

- Increase Bandwidth

- Multiple access ports combine into one logical link.

- Redundancy

- If one link goes down, traffic continues through remaining links.

- STP Optimization

- STP sees Port-Channel as one link → no unnecessary blocking.

- Simplified Management

- Configure VLAN, STP on Port-Channel interface instead of each physical port.

✅ Advantages

- Higher throughput

- Redundancy

- No STP blocking per link

- Easier VLAN management

- Supports dynamic/static bundling (LACP/PAgP or ON)

❌ Disadvantages

- All member ports must have exact same configuration (speed, duplex, VLAN)

- Load balancing might not be perfect

- Wrong configuration can create loops

- Member links must be same type and speed

Configuration:

en

config t

hostname INBOM25DS01

vlan 10

name PC_VLAN

exit

interface range Fa0/1 - 4

description *Connected to INBOM25DS02 for EtherChannel*

switchport mode access

switchport access vlan 10

no shutdown

channel-group 2 mode active

exit

interface port-channel 2

switchport mode access

switchport access vlan 10

no shutdown

exit

===================================================

en

config t

hostname INBOM25DS02

vlan 10

name PC_VLAN

exit

interface range Fa0/1 - 4

description *Connected to INBOM25DS01 for EtherChannel*

switchport mode access

switchport access vlan 10

no shutdown

channel-group 2 mode active

exit

interface port-channel 2

switchport mode access

switchport access vlan 10

no shutdown

exit