37. Configure EtherChannel with trunk ports

What is EtherChannel?

EtherChannel is a technology on Cisco switches that bundles multiple physical links into one logical interface called a Port-Channel.

Example:
4 physical 1-Gbps links → 1 logical 4-Gbps link.

✅ Why do we use EtherChannel (with trunk ports)?

We use EtherChannel mainly for two reasons:

1. Increase Bandwidth

Multiple links combine to give higher speed.

2. Redundancy

If one link fails, the Port-Channel stays UP using remaining links.

3. STP Optimization

STP sees the Port-Channel as one link →
No blocking of individual links → Better utilization.

4. Load Balancing

Traffic is distributed across member links automatically.

5. Trunking Across Switches

EtherChannel trunk allows multiple VLANs to travel across switches with redundancy + bandwidth.

✅ Advantages of EtherChannel (Especially Trunk)

✔ 1. Increased throughput

4 x 1G links = 4G combined capacity.

✔ 2. Redundancy

If one link fails, communication continues.

✔ 3. No STP blocking

STP treats Port-Channel as one interface.

✔ 4. Easy management

Configure VLANs, trunks, STP on Port-Channel (one interface), not on each physical port.

✔ 5. Supports LACP/PAgP or Static

Flexible options for dynamic/static bundling.

❌ Disadvantages of EtherChannel

✘ 1. All ports must match exactly

Speed, duplex, trunk/native VLAN, allowed VLANs…
If mismatch → "Ports not compatible" → bundle will fail.

✘ 2. Load-balancing may not be perfect

Traffic from one source-destination pair may stick to one link.

✘ 3. Incorrect configuration can cause loops

If mismatched EtherChannel modes on both ends.

✘ 4. Member links must be of same type

Same speed, same model (recommended).

Configuration:

enable

configure terminal

Hostname INBOM25CS01

! Select physical interfaces

interface range gi1/0/1 - 2

description *Connected to INBOM25CS02

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode active   ! (LACP)

exit

! Configure Port-Channel

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

ex

----------------------------------------

enable

configure terminal

Hostname INBOM25CS02

! Select physical interfaces

interface range gi1/0/1 - 2

description *Connected to INBOM25CS01

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode active

exit

! Configure Port-Channel

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

ex