39_Configure EtherChannel with STP

What is EtherChannel with STP?

- EtherChannel: Combines multiple physical links into one logical link (Port-Channel) between switches.

- STP (Spanning Tree Protocol): Protocol that prevents loops in a network with redundant links.

- EtherChannel + STP: The logical Port-Channel is treated as a single interface by STP, so multiple physical links do not create loops.

In short: Bundled links + loop-prevention mechanism = efficient and safe network topology.

2️⃣ Why do we use EtherChannel with STP?

- Avoid STP blocking on redundant links

  - Normally, STP blocks redundant links to prevent loops.

  - With EtherChannel, all member links carry traffic while STP sees it as one link.

- Increase bandwidth with redundancy

  - Traffic load can be shared among bundled links without STP blocking some of them.

- Simplify STP management

  - STP configuration is applied on Port-Channel, not each individual link.

- Predictable network topology

  - Combine with STP root bridge priority for controlled path selection.

3️⃣ Advantages of EtherChannel with STP

- Higher throughput: Multiple links act as a single logical link.

- Redundancy: If one link fails, the Port-Channel stays up.

- STP optimization: Reduces blocked links → all links carry traffic.

- Simpler management: Configure STP once on Port-Channel, not on each physical port.

- Loop prevention: STP ensures no network loops occur.

4️⃣ Disadvantages / Limitations

- Misconfiguration risk: Wrong STP priority or inconsistent EtherChannel config can cause loops or downed links.

- Load balancing is limited: Traffic between a single source-destination pair may still use only one physical link.

- Switch compatibility: All member links must match exactly (speed, duplex, VLAN, STP version).

- Troubleshooting complexity: If something goes wrong, it’s sometimes harder to pinpoint whether it’s STP or EtherChannel issue.

LAB topology

SW1 Gi1/0/1-2  <---->  Gi1/0/1-2 SW2

VLAN 10

Port-Channel 1 (LACP)

SW1 as STP root

Configuration:

1.Connected 2 L2 switch and perfomed LACP + STP via allowing trunk port allowing all vlan

en

config t

hostname INBOM25L201

vlan 10

name PC_VLAN

ex

spanning-tree vlan 10 priority 4096

! Configure physical interfaces for trunk EtherChannel

interface range fa0/1 - 2

description *Connected to INBOM25L202 trunk *

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

===================================

en

config t

hostname INBOM25L202

vlan 10

name PC_VLAN

ex

! Configure physical interfaces for trunk EtherChannel

interface range fa0/1 - 2

description *Connected to INBOM25L201 trunk *

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

=========================================================================

2.Connected 2 L2 switch and perfomed LACP + STP via allowing trunk port allowing vlan 10

en

config t

hostname INBOM25L203

vlan 10

name PC_VLAN

ex

spanning-tree vlan 10 priority 4096

! Configure physical interfaces for trunk EtherChannel

interface range fa0/1 - 2

description *Connected to INBOM25L204 trunk with specific vlan10*

switchport mode trunk

switchport trunk allowed vlan 10

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan 10

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

===================================

en

config t

hostname INBOM25L204

vlan 10

name PC_VLAN

ex

! Configure physical interfaces for trunk EtherChannel

interface range fa0/1 - 2

description *Connected to INBOM25L203 trunk with specific vlan10*

switchport mode trunk

switchport trunk allowed vlan 10

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan 10

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

=======================================================================

3.Connected 2 L2 switch and perfomed LACP + STP via allowing access port allowing vlan 10

en

config t

hostname INBOM25L205

vlan 10

name PC_VLAN

ex

spanning-tree vlan 10 priority 4096

! Configure physical interfaces for trunk EtherChannel

interface range fa0/1 - 2

description *Connected to INBOM25L206 access with specific vlan10*

switchport mode access

switchport access vlan 10

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode access

switchport access vlan 10

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

===================================

config t

hostname INBOM25L206

vlan 10

name PC_VLAN

ex

! Configure physical interfaces for trunk EtherChannel

interface range fa0/1 - 2

description *Connected to INBOM25L205 access with specific vlan10*

switchport mode access

switchport access vlan 10

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode access

switchport access vlan 10

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

================================================================================

4.Connected 2 L3 switch and perfomed LACP + STP via allowing trunk port allowing all vlan

en

config t

hostname INBOM25L301

vlan 10

name PC_VLAN

ex

spanning-tree vlan 10 priority 4096

! Configure physical interfaces for trunk EtherChannel

interface range Gi1/0/1 - 2

description *Connected to INBOM25L302 trunk *

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

===================================

en

config t

hostname INBOM25L302

vlan 10

name PC_VLAN

ex

! Configure physical interfaces for trunk EtherChannel

interface range Gi1/0/1 - 2

description *Connected to INBOM25L301 trunk *

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

channel-group 1 mode active

exit

! Configure Port-Channel interface

interface port-channel 1

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

! Configure SVI for VLAN 10

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

================================================================================

Working for all the above but 
for Specific trunk and access port l3 switch will be not working