40. Configure EtherChannel redundancy test

What is EtherChannel Redundancy?

EtherChannel redundancy means:

If one physical link inside an EtherChannel goes down, traffic continues to flow using the remaining links — without downtime, without reconvergence.

EtherChannel = multiple physical links combined into one logical link (Port-Channel).

If any link fails → Port-Channel stays UP → network stays reachable → zero interruption.

Why Do We Do This?

We perform EtherChannel redundancy to:

Avoid link failure outages

Without EtherChannel, if a single cable goes down → STP blocks failover → traffic drops for 30–50 seconds.

With EtherChannel → failover is instant, no spanning-tree reconvergence.

Increase reliability

Even if 1 out of 4 links fail, EtherChannel continues to forward traffic.

Maintain consistent bandwidth

If one link goes down, only bandwidth reduces, but connectivity remains.

Test high availability

We intentionally disconnect cables to ensure network stays stable.

3️⃣ Advantages

High Availability

- Traffic flow continues even if 1 or more physical links fail.

Faster Failover

- No STP recalculation.

- Failover time = milliseconds.

Load Balancing

- Traffic is spread across multiple links (depending on hashing algorithm).

Simplified STP

- STP treats the whole EtherChannel as one logical link, reducing complexity.

More Bandwidth

- 2 links = double

- 4 links = 4x bandwidth

- IISL designer friendly

4️⃣ Disadvantages

Configuration Mistakes Cause Bundle Failure

If speed, duplex, VLANs, modes mismatch → ports won't join Port-Channel → down time.

Bandwidth is NOT perfectly combined

Load balancing is hash-based
→ not all flows use all links equally.

Troubleshooting can be harder

Need to check:

- physical links

- LACP/PAgP

- hashing rules

- VLAN consistency

Misconfiguration = STP loops

If one side is configured wrong → loops can occur.

Hardware limited

Some switches support only:

- 4 links per port-channel

- 8 links per port-channel

Configuration:
enable

configure terminal

hostname INBOM25CS01

! Create VLAN

vlan 10

name PC_VLAN

exit

! SVI for VLAN 10

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! Clean interfaces

default interface gi1/0/1

default interface gi1/0/2

! Configure physical interfaces

interface range gi1/0/1 - 2

description *EtherChannel to INBOM25CS02*

switchport

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode active

no shutdown

exit

! Configure Port-Channel

interface port-channel 1

description *LACP EtherChannel to INBOM25CS02*

switchport

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

======================================================

enable

configure terminal

hostname INBOM25CS02

! Create VLAN

vlan 10

name PC_VLAN

exit

! SVI for VLAN 10

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

! Clean interfaces

default interface gi1/0/1

default interface gi1/0/2

! Configure physical interfaces

interface range gi1/0/1 - 2

description *EtherChannel to INBOM25CS01*

switchport

switchport mode trunk

switchport trunk allowed vlan all

channel-group 1 mode active

no shutdown

exit

! Configure Port-Channel

interface port-channel 1

description *LACP EtherChannel to INBOM25CS01*

switchport

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit