1️⃣ What is Port Security (Static MAC)?

Point-wise:

- A switch security feature that limits which MAC address can access a port.

- You manually configure the specific MAC addresses allowed on that port.

- If any other device connects → switch triggers a violation.

- Helps prevent unknown, unauthorized, or malicious devices from joining the network.

- Works only on access ports (not on trunk ports).

- Has 3 violation modes: protect, restrict, shutdown.

- Commonly used in offices, servers, CCTV, printers, etc.

2️⃣ Why do we configure Static MAC Port Security?

Point-wise:

- To block unauthorized devices.

- To stop MAC-flooding attacks.

- To ensure only known, approved devices connect.

- To protect network from users connecting rogue switches/hubs.

- To maintain strict device-to-port mapping.

- To increase security without requiring complex authentication systems like 802.1X.

- To prevent network changes in sensitive areas (server rooms, backbone).

- To ensure predictable and controlled traffic.

3️⃣ Advantages (Pros)

Point-wise:

- High security — only configured MACs allowed.

- Prevents unauthorized access.

- Protects against MAC table overflow/flooding attacks.

- Ensures device consistency per port.

- Blocks users from connecting their own devices.

- Reduces risk of loops caused by unmanaged switches/hubs.

- Good for IoT, CCTV, printers, where MAC never changes.

- Helps in asset and port tracking.

4️⃣ Disadvantages (Cons)

Point-wise:

- High administrative work — MACs must be manually configured.

- When a user changes laptop/PC → new MAC not allowed → port shuts down.

- Not scalable in large networks.

- Can cause accidental port shutdowns if MAC is wrong.

- Can be bypassed by MAC spoofing (attacker fakes allowed MAC).

- Does not protect data — no encryption.

- Troubleshooting increases because ports often go into err-disable mode.

Configuration:

enable

configure terminal

hostname SW1

! Step 1: Make port access port

interface fa0/1

switchport mode access

switchport access vlan 10

! Step 2: Enable port security

switchport port-security

! Step 3: Allow ONLY 1 MAC address on this port

switchport port-security maximum 1

! Step 4: Configure STATIC allowed MAC address

switchport port-security mac-address 00d0.ba11.2233

(replace with your PC MAC)

! Step 5: Violation mode (recommended: shutdown)

switchport port-security violation shutdown

! Step 6: Enable port

no shutdown

exit

verification:

Test 1: Connect correct PC

✔ Link works
✔ Ping works
✔ No violation

Test 2: Disconnect PC & connect another one

❌ Port will go err-disabled
❌ Ping fails
❌ "Link down" shows

en

config t

hostname INBOM25CS01

vlan 10

name wired_PC

ex

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ---- DHCP POOLS ----

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

! Access ports

interface range gi1/0/1 - 2

switchport mode access

switchport access vlan 10

no shutdown

exit

! port security

interface gi1/0/1

switchport port-security

switchport port-security mac-address 0030.F243.D902

switchport port-security maximum 1

switchport port-security violation restrict

no shutdown

ex

interface gi1/0/2

switchport port-security

switchport port-security mac-address 0090.2187.55D7

switchport port-security maximum 1

switchport port-security violation shutdown

no shutdown

ex

ex

Working

Failed when another connected

S