42_Configure port security (sticky MAC)

1️⃣ What is Port Security (Sticky MAC)?

Sticky MAC is a feature of port security where the switch automatically learns the MAC address of connected devices and stores them as secure addresses.

- Unlike static MAC, you don’t need to manually type the MAC.

- The switch “remembers” the MAC and keeps it in running-config (can be saved in startup-config).

- If another device connects → violation occurs based on configured mode.

2️⃣ Why do we use Sticky MAC?

- Simplifies administration — no need to manually enter MACs

- Still restricts unauthorized devices

- Useful for dynamic environments where new devices join frequently

- Can automatically learn PCs, printers, phones

3️⃣ Advantages

- Automatic MAC learning → reduces manual work

- Prevents unauthorized access → security is maintained

- Can be saved in startup-config → persistent after reboot

- Works well in offices where PC MACs change frequently

4️⃣ Disadvantages

- If a PC is replaced → old MAC must be removed manually

- Sticky MAC learning may fill up if many devices connect

- Still vulnerable to MAC spoofing (rare)

- Slightly more complex than pure static MAC

Configuration:

en

config t

hostname INBOM25CS01

vlan 10

name wired_PC

ex

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ---- DHCP POOLS ----

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

! Access ports

interface range gi1/0/1 - 2

switchport mode access

switchport access vlan 10

no shutdown

exit

! port security

interface gi1/0/1

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 1

switchport port-security violation restrict

no shutdown

ex

interface gi1/0/2

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 1

switchport port-security violation shutdown

no shutdown

ex

ex

Before: