43_Configure port security violation modes (shutdown/restrict/protect)

What are Port Security Violation Modes?

Port Security violation mode determines what the switch does when an unauthorized device (MAC) connects to a port.

There are 3 violation modes:

| Mode | Behavior | Notes |
| --- | --- | --- |
| shutdown | Port goes err-disabled. Logs a violation. Most secure. | Default mode. Admin must recover port manually. |
| restrict | Drops unauthorized traffic. Logs a violation. Does NOT shut the port. | Useful in production; keeps port up. |
| protect | Drops unauthorized traffic silently. No logs, no shutdown. | Least intrusive; may go unnoticed. |

🔵 2️⃣ Why Do We Use Violation Modes?

- To enforce network security

- To prevent unauthorized devices from using the network

- To control behavior depending on business requirements:

  - High-security → shutdown

  - Medium-security → restrict

  - Low-security / guest ports → protect

🔵 3️⃣ Advantages & Disadvantages

Advantages

- Protects your network from rogue devices

- Reduces MAC flooding attacks

- Can adapt behavior based on need (shutdown/restrict/protect)

Disadvantages

- Shutdown: Manual intervention needed

- Restrict: Unauthorized users still try to connect → logs only

- Protect: No alerts → hard to notice violation

Configuration:

en

config t

hostname INBOM25CS01

vlan 10

name wired_PC

ex

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ---- DHCP POOLS ----

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

! Access ports

interface range gi1/0/1 - 3

switchport mode access

switchport access vlan 10

no shutdown

exit

! port security

interface gi1/0/1

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 1

switchport port-security violation restrict

no shutdown

ex

interface gi1/0/2

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 1

switchport port-security violation shutdown

no shutdown

ex

interface gi1/0/3

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 1

switchport port-security violation protect

no shutdown

ex

ex