44_Configure maximum MAC addresses per port

1️⃣ What is “Maximum MAC Addresses per Port”?

- This feature limits how many MAC addresses can be learned or allowed on a single switch port.

- Helps control access and prevent MAC flooding attacks.

Default maximum = 1 (most Cisco switches).

🔵 2️⃣ Why Do We Use Maximum MAC Addresses?

- Security: Prevent rogue devices from connecting

- Control: Limit number of devices per port (e.g., PC + IP phone)

- Prevent MAC table overflow: Stops MAC flooding attacks

🔵 3️⃣ Advantages

- Prevents unauthorized devices from connecting

- Helps enforce port security policies

- Can allow multiple devices safely (like PC + phone)

- Reduces risk of network attacks

🔵 4️⃣ Disadvantages

- Too low → legitimate devices may be blocked

- Requires monitoring if multiple devices need the same port

- Static allocation can be hard to manage in large networks

Configuration:

en

config t

hostname INBOM25CS01

vlan 10

name wired_PC

ex

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ---- DHCP POOLS ----

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

! Access ports

interface gi1/0/1

switchport mode access

switchport access vlan 10

no shutdown

exit

! port security

interface gi1/0/1

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 2

switchport port-security violation shutdown

no shutdown

ex

ex

wr mem

cop running-config startup-config