45_Configure aging for sticky MAC

1️⃣ What is Sticky MAC Aging?

- Sticky MAC aging allows the switch to automatically remove learned sticky MAC addresses after a certain period of time.

- This is useful in dynamic environments where devices connect and disconnect frequently.

Without aging → learned sticky MAC stays forever unless manually removed.

2️⃣ Why Do We Use Sticky MAC Aging?

- To free up the port for new devices

- To reduce manual MAC removal

- To avoid port being locked if a device is replaced or moved

- Helps dynamic office setups (hot desks, guest PCs, phones)

3️⃣ Types of Aging

There are two types:

| Type | Meaning |
| --- | --- |
| Absolute | Sticky MAC is removed after a fixed timer, regardless of activity. |
| Inactivity | Sticky MAC is removed only if no traffic is seen from that MAC for the timer duration. |

4️⃣ Advantages

- Flexible → port adapts to new devices

- Reduces administrative work

- Keeps network secure but dynamic

- Works well in hot-desk or shared-office environments

5️⃣ Disadvantages

- Requires monitoring → MACs can age out unexpectedly

- May cause temporary access loss if timer is too short

- Slightly more complex than static sticky MAC

Configuration:

en

config t

hostname INBOM25CS01

vlan 10

name wired_PC

ex

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ---- DHCP POOLS ----

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

! Access ports

interface range gi1/0/1-2

switchport mode access

switchport access vlan 10

no shutdown

exit

! port security

interface gi1/0/1

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation shutdown

switchport port-security aging time 2

!below command is not supported in packet tracer

switchport port-security aging type absolute

no shutdown

ex

interface gi1/0/2

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation shutdown

switchport port-security aging time 2

!below command is not supported in packet tracer

switchport port-security aging type inactivity

no shutdown

ex

ex

wr mem

copy running-config startup-config