50_Configure switchport security for unused ports

1. What is Switchport Security for Unused Ports?

Switchport security for unused ports is a security best practice where all unused (not connected) switch interfaces are:

- Disabled (shutdown)

- or placed in a “blackhole” VLAN

- and configured as static access ports

- with port security features enabled (optional)

This prevents unauthorized users from plugging into free Ethernet ports and accessing the network.

In short:

If a port is not being used → Turn it off or secure it.

2. Why Do We Do This? (Purpose)

Unused ports are a major security risk. Anyone can:

- Physically walk into office

- Plug a laptop into an unused LAN port

- Get access to internal network

Switchport security blocks such unauthorized access.

We do this to:

Prevent unauthorized access to internal network

No free port should work automatically.

Prevent rogue DHCP servers

Someone connecting a laptop running DHCP can disturb your network.

Prevent VLAN hopping

Unused ports should not be in production VLANs.

Reduce attack surface

Less attack points = better network security.

Maintain network hygiene

Organized port allocation and secure design.

3. Advantages of Securing Unused Ports

1. Highest Physical Security

Stops anyone from plugging in without permission.

2. Prevents Unauthorized Devices

Unknown MAC addresses cannot connect.

3. Protects Against Rogue DHCP Servers

Unauthorized DHCP offers are blocked.

4. Prevents VLAN Access

Unused ports moved to blackhole VLAN → no network access.

5. Limits Attack Vectors

Port security prevents:

- MAC flooding attacks

- CAM table overflow

- DHCP attacks

- Spoofing attacks

6. Helps Pass Security Audits

Most compliance standards (PCI-DSS, ISO, SOC2) require unused ports to be disabled.

4. Disadvantages / Limitations

1. Extra Operational Effort

Every time someone needs that port → you must manually enable it.

2. Troubleshooting Confusion

Engineers may forget ports are “shutdown” → spend time troubleshooting.

3. Misconfiguration Risk

Wrong use of port-security can:

- Shutdown important ports

- Lock out legitimate devices

4. Requires Documentation

You must maintain a record of which ports are active/unused.

5. Sticky MAC Can Cause Issues

Moving devices between ports requires clearing sticky MAC addresses.

Configuration:

enable

configure terminal

! ------------------------------

! Hostname

! ------------------------------

hostname INBOM25AS01

! ------------------------------

! Create Blackhole VLAN

! ------------------------------

vlan 999

name UNUSED_PORTS

exit

! ------------------------------

! Secure all unused ports

! Example: Gi1/0/7–24

! ------------------------------

interface range gi1/0/7 - 24

switchport mode access

switchport access vlan 999

switchport nonegotiate

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

spanning-tree portfast

shutdown

exit

vlan 10

name USERS

exit

!-- SVI interfaces

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ---- DHCP POOLS ----

ip dhcp pool SALES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

! Excluded addresses

ip dhcp excluded-address 192.168.10.1

interface range Gig1/0/1-6

switchport mode access

switchport access vlan 10

spanning-tree portfast

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

no shutdown

exit

ex

write memory

copy running-config startup-config