52_Configure SSH for secure remote access

1. What is SSH?

SSH (Secure Shell) is a protocol that allows secure remote access to network devices such as switches and routers.

- Encrypts all traffic (username, password, commands)

- Replaces insecure protocols like Telnet

- Works over TCP port 22

2. Why Do We Use SSH?

We use SSH to:

- Secure remote device management – Admins can manage switches securely over the network.

- Encrypt sensitive data – Prevents password sniffing or command interception.

- Replace Telnet – Telnet sends credentials in plain text (unsafe).

- Enable automation and scripting – SSH is widely supported for automated scripts.

3. Advantages of SSH

| Advantage | Explanation |
| --- | --- |
| Secure | Encrypts traffic, protects credentials and commands |
| Remote Access | Admins can configure switches from anywhere |
| Authentication Options | Supports local users or AAA server (RADIUS/TACACS+) |
| Audit & Logging | Commands can be logged securely |
| Better than Telnet | No plaintext vulnerabilities |

4. Disadvantages / Limitations

| Disadvantage | Explanation |
| --- | --- |
| Requires CPU & Memory | Encryption adds overhead |
| Requires Proper Time/Date | SSH certificates may fail if clock is wrong |
| Needs Initial Setup | Device must have hostname, domain, and crypto keys configured |
| Cannot Access if IP is Missing | Switch must have IP on VLAN or SVI |
| Older Devices May Not Support SSH v2 | Must check device IOS version |

Configuration:

enable

configure terminal

! -----------------------------

! Hostname & Domain (for SSH)

! ------------------------------

hostname INBOMCS01

ip domain-name adityalab.com

! ------------------------------

! Create User VLAN and SVI

! ------------------------------

vlan 10

name USERS

exit

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ------------------------------

! Configure DHCP Pool for User VLAN

! ------------------------------

ip dhcp excluded-address 192.168.10.1 192.168.10.49

ip dhcp excluded-address 192.168.10.101 192.168.10.110

ip dhcp pool USERS_POOL

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

! ------------------------------

! Assign VLAN 10 to User Ports (Gi1/0/1–24)

! ------------------------------

interface range gi1/0/1 - 24

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

! ------------------------------

! Configure SSH for Secure Remote Access

! ------------------------------

username admin privilege 15 secret Cisco123

ip ssh version 2

crypto key generate rsa general-keys modulus 2048

! Enable VTY lines for SSH only

line vty 0 4

login local

transport input ssh

exit

! ------------------------------

! Optional: Logging and Banner

! ------------------------------

banner motd # Unauthorized Access Prohibited! #

service timestamps log datetime msec

! ------------------------------

! Save configuration

! ------------------------------

end

write memory