53_Disable Telnet and enable SSH only

1. What is Telnet and Why Disable It?

- Telnet is a legacy protocol for remote access to network devices.

- It sends credentials and commands in plain text → not secure.

- Modern networks use SSH for encrypted remote management.

Why disable Telnet:

- Prevents credential sniffing over the network.

- Enforces secure administration policy.

2. Why Enable SSH Only?

- SSH encrypts all traffic (commands + credentials).

- Provides secure remote access to switches and routers.

- Supports local user accounts or AAA authentication.

3. Advantages of SSH over Telnet

| Advantage | Explanation |
| --- | --- |
| Encrypted Traffic | Prevents hackers from seeing passwords or commands |
| Secure Remote Access | Admins can manage devices safely over WAN/LAN |
| Authentication Options | Local users, RADIUS, TACACS+ |
| Logging & Audit | Commands can be logged securely |
| Modern Standard | Required by most security policies |

4. Disadvantages / Considerations

| Disadvantage | Explanation |
| --- | --- |
| CPU Overhead | Encryption consumes more resources |
| Initial Setup Required | Hostname, domain, RSA keys, and user account needed |
| Cannot Fall Back to Telnet | Legacy scripts using Telnet will fail |
| Time Sensitive | SSH certificates may fail if switch clock is wrong |

Configuration:

enable

configure terminal

! ------------------------------

! Hostname & Domain (for SSH)

! ------------------------------

hostname SWITCH1

ip domain-name adityalab.com

! ------------------------------

! Create VLANs

! ------------------------------

vlan 10

name USERS

! ------------------------------

! Configure SVI for VLAN 10 (Switch Management IP)

! ------------------------------

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ------------------------------

! Assign VLAN 10 to user ports (Gi1/0/1–5)

! ------------------------------

interface range gi1/0/1 - 5

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

! ------------------------------

! Configure SSH (Disable Telnet)

! ------------------------------

username admin privilege 15 secret Cisco123

ip ssh version 2

crypto key generate rsa general-keys modulus 2048

line vty 0 4

login local

transport input ssh

exit

! ------------------------------

! Optional Banner and Logging

! ------------------------------

banner motd # Unauthorized Access Prohibited! #

service timestamps log datetime msec

! ------------------------------

! Save Configuration

! ------------------------------

ex

write memory

copy running-config startup-config