54_Configure login banners

Configuration:

1. What is a Login Banner?

A banner is a text message displayed to anyone who tries to access the switch (locally or remotely).

Types of banners:

| Type | When Displayed | Example Use |
| --- | --- | --- |
| MOTD (Message of the Day) | On login, before username/password | Security warning, company policy |
| Login Banner | Right before login prompt | Legal notice or warning |
| Exec Banner | After login, in privileged mode | Instructions or info |

The most common is MOTD banner.

2. Why Use a Login Banner?

- Warn unauthorized users they are accessing a secured network device.

- Display company policies or legal disclaimers.

- Deter casual intrusion attempts.

3. Advantages

- Improves security awareness.

- Acts as a legal notice for unauthorized access.

- Customizable for instructions or contact info.

4. Disadvantages

- Does not prevent unauthorized access (only a warning).

- Long banners may be annoying for frequent users.

- Needs careful formatting to avoid display issues.

Configuration:

enable

configure terminal

! ------------------------------

! Hostname & Domain (for SSH)

! ------------------------------

hostname SWITCH1

ip domain-name adityalab.com

! ------------------------------

! Create VLANs

! ------------------------------

vlan 10

name USERS

! ------------------------------

! Configure SVI for VLAN 10 (Switch Management IP)

! ------------------------------

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

! ------------------------------

! Assign VLAN 10 to user ports (Gi1/0/1–5)

! ------------------------------

interface range gi1/0/1 - 5

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

! ------------------------------

! Configure SSH (Disable Telnet)

! ------------------------------

username admin privilege 15 secret Cisco123

ip ssh version 2

crypto key generate rsa general-keys modulus 2048

line vty 0 4

login local

transport input ssh

exit

! ------------------------------

! Configure MOTD Banner

! ------------------------------

banner motd #

****************************************************************

*    WARNING: Unauthorized access to this device is prohibited!  *

*    All activities are logged.                                 *

*    If you are not authorized, disconnect immediately.         *

****************************************************************

#

! ------------------------------

! Optional: Login Banner (before username prompt)

! ------------------------------

banner login #

****************************

*    LOGIN TO SWITCH ONLY   *

*    Authorized personnel only *

****************************

#

!The below was not working in the packet tracer

! ------------------------------

! Optional: Exec Banner (after privileged login)

! ------------------------------

banner exec #

******************************************

*    You are now in privileged mode      *

*    Use commands responsibly             *

******************************************

#

ex

write memory

copy running-config startup-config