57_Configure VTP pruning

1. What is VTP Pruning?

When VLANs are created in a network, switches flood broadcast, multicast, and unknown unicast traffic to all trunk ports—even if the destination switch has no ports in that VLAN.

VTP Pruning helps stop unnecessary broadcast flooding by restricting VLAN traffic on trunks only to switches that actually need it.

In simple words:

➡️ “Send VLAN traffic only where it is needed—don’t waste trunk bandwidth.”

2. Why Do We Use VTP Pruning?

Because in large networks, many VLANs exist but not all switches have hosts in those VLANs.

Example:

- VLAN 10 exists on Switch A and B

- VLAN 20 exists on Switch A and C

Without pruning:
→ VLAN 20 broadcasts are also sent to Switch B (waste)

With pruning:
→ VLAN 20 traffic stays only between A and C

3. How VTP Pruning Works (Short & Clear)

- Switch advertises which VLANs exist on which trunk.

- If a switch does not have any active ports in VLAN X → It tells upstream switch:
“Do not send VLAN X traffic to me.”

- Trunk is pruned for that VLAN.

⭐ 4. Advantages of VTP Pruning

✔ 1. Saves trunk bandwidth

Unnecessary broadcasts/multicasts reduced.

✔ 2. Reduces CPU usage on switches

Switch does not process unwanted broadcast frames.

✔ 3. Prevents unnecessary flooding in large networks

✔ 4. Faster network performance

✔ 5. Reduces broadcast storms impact

❌ 5. Disadvantages of VTP Pruning

✔ 1. Works only in VTP Server/Client mode

Pruning is NOT supported in Transparent mode.

✔ 2. Can cause connectivity issues if not designed properly

If VLAN gets pruned incorrectly, devices lose communication.

✔ 3. Not recommended for MPLS/VPLS / Complex topologies

✔ 4. Requires VTP v1 or v2 (not v3)

(Though v3 supports pruning control)

Configuration

enable

configure terminal

hostname SW1

! -------------------------

! VTP CONFIGURATION (SERVER)

! -------------------------

vtp domain ADITYA

vtp mode server

vtp password cisco123

vtp pruning

! Create VLANs (Server only)

vlan 10

name VLAN10

ex

vlan 20

name VLAN20

ex

vlan 30

name VLAN30

exit

! -------------------------

! TRUNK PORTS

! -------------------------

interface range gi1/0/1 - 2

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

ex

write mem

copy running-config startup-config

=========================================================

enable

configure terminal

hostname INBOM25DS01

! -------------------------

! VTP CONFIGURATION (CLIENT)

! -------------------------

vtp domain ADITYA

vtp mode client

vtp password Cisco123

! -------------------------

! TRUNK PORTS

! -------------------------

interface range gi1/0/1 - 2

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

ex

write mem

copy running-config startup-config

=============================================================

enable

configure terminal

hostname INBOM25DS02

! -------------------------

! VTP CONFIGURATION (CLIENT)

! -------------------------

vtp domain ADITYA

vtp mode client

vtp password Cisco123

! -------------------------

! TRUNK PORTS

! -------------------------

interface range gi1/0/1 - 2

switchport mode trunk

switchport trunk allowed vlan all

no shutdown

exit

ex

write mem

copy running-config startup-config

Command for VTP pruning didn’t work on this Packet tracket switch series