60_Configure Private VLANs

1. What Are Private VLANs (PVLANs)?

Private VLANs (PVLANs) are an advanced feature that allow you to break a single VLAN into smaller isolated segments to control communication between devices within the same VLAN.

Think of it like:

One big VLAN → divided into isolated small sub-VLANs.

They provide Layer 2 isolation while still using one IP subnet.

🔵 2. Why Do We Use Private VLANs? (Purpose)

We use PVLANs when:

✔ You want hosts to be in the same IP network

BUT

✔ You do NOT want them to talk to each other directly.

This is used to increase security without using many VLANs + subnets.

Typical use cases:

- Internet-facing servers in a DMZ

- ISP shared networks (Hotels, Airports, Public WiFi)

- Data centers hosting multiple customer servers

- Preventing malware spread internally

- Protecting sensitive servers from lateral movement

🔵 3. Types of PVLANs (Very Important)

Private VLANs consist of three types:

A. Primary VLAN

- Parent VLAN

- All secondary VLANs belong to this

B. Secondary VLANs

These are of 2 types:

1. Isolated VLAN

- Host cannot talk to ANY other host, even in same isolated VLAN

- Can ONLY talk to the promiscuous port (router/firewall)

2. Community VLAN

- Hosts inside this VLAN can talk to each other

- But cannot talk to other community VLANs

- Can talk to the promiscuous port

🔵 4. Types of PVLAN Ports

| Port Type | Behavior |
| --- | --- |
| Promiscuous Port | Can talk to ALL VLANs |
| Isolated Port | Can talk only to promiscuous port |
| Community Port | Can talk inside community + promiscuous |

🔵 5. Advantages of Private VLANs

✔ 1. Strong security at Layer 2

Prevents host-to-host communication inside the same subnet.

✔ 2. Saves IP addresses

No need to create separate subnets/VLANs for isolation.

✔ 3. Prevents ARP spoofing / MITM attacks

Isolated hosts cannot ARP each other.

✔ 4. Reduces broadcast domain

Broadcasts from isolated ports do not reach other isolated ports.

✔ 5. Best for large shared networks

Used in:

- Hotels

- ISPs

- Public Wi-Fi

- Campus networks

✔ 6. Very useful in DMZ

Protect servers even if sitting in the same DMZ subnet.

🔵 6. Disadvantages of Private VLANs

❌ 1. Not supported on all switches

Only some L3/L2 switches can do PVLAN (e.g., 3560, 3750, 4500, 6500).
NOT supported in Packet Tracer on all models.

❌ 2. Configuration is complex

Multiple VLANs, mappings, and port types must be correct.

❌ 3. Troubleshooting is harder

Traffic flows change based on port type (promiscuous / isolated / community).

❌ 4. Works only in same Layer-2 domain

Does not protect against traffic routed through L3.

❌ 5. Cannot use with some features

Some switches block:

- VTP

- SPAN

- DAI

- DHCP Snooping
with PVLANs.

❌ 6. Requires L3 device for routing

Promiscuous port must connect to:

- Router-on-stick

- Firewall

- L3 SVI

1. What is a Private VLAN?

- Private VLAN (PVLAN) is an extension of the VLAN concept.

- It segregates traffic within a VLAN to provide layer 2 isolation between ports while still allowing communication with a common uplink (promiscuous port).

- Used mainly in service provider networks, data centers, and hosting environments.

2. Types of Private VLANs

- Primary VLAN – The main VLAN that contains all secondary VLANs.

- Secondary VLANs – Subdivided VLANs inside the primary VLAN:

  - Isolated VLAN: Ports cannot communicate with other ports in the same VLAN; can only communicate with promiscuous ports.

  - Community VLAN: Ports can communicate with other ports in the same community VLAN and promiscuous ports but not with ports in other community VLANs.

- Promiscuous Port: Can communicate with all PVLAN ports (used for router/gateway or server uplinks).

3. Why Do We Use Private VLANs?

- Security: Isolates devices in the same VLAN to prevent unwanted traffic between them.

- Simplifies Network Design: One VLAN can host multiple isolated networks without using multiple VLAN IDs.

- Efficient IP Utilization: Reduces the number of VLANs required for isolation in large networks.

- Control Traffic Flow: Provides fine-grained Layer 2 traffic control.

4. Advantages of Private VLANs

- Enhanced Security: Devices cannot snoop or communicate with other devices in the same VLAN unless allowed.

- Efficient VLAN Usage: Supports multiple isolated groups in a single VLAN.

- Supports Service Provider Designs: Commonly used in data centers and hosting networks.

- Flexible Traffic Control: Promiscuous, isolated, and community port types allow controlled communication.

5. Disadvantages of Private VLANs

- Configuration Complexity: More complex than standard VLANs.

- Compatibility Limitations: Not all switches or older devices support PVLANs.

- Troubleshooting Difficulty: Layer 2 isolation can cause confusion if not documented well.

- Spanning Tree Considerations: PVLANs need careful STP design to avoid loops.

6. Key Points

- PVLANs are always associated with a primary VLAN.

- Promiscuous ports connect to routers, servers, or uplinks.

- Community ports can communicate within their group.

- Isolated ports cannot communicate with any other secondary VLAN port, only with promiscuous ports.

- Commands differ slightly between Cisco IOS versions, but the concept remains the same.

Configuration:
The configuration is not supported in packet tracer

enable

configure terminal

vlan 100

private-vlan primary

vlan 101

private-vlan isolated

vlan 102

private-vlan community

vlan 100

private-vlan association 101,102

exit

interface fa0/1

switchport mode private-vlan promiscuous

switchport private-vlan mapping 100 101,102

no shutdown

exit

interface fa0/2

switchport mode private-vlan host

switchport private-vlan host-association 100 101

no shutdown

exit

interface fa0/3

switchport mode private-vlan host

switchport private-vlan host-association 100 102

no shutdown

exit

interface fa0/4

switchport mode private-vlan host

switchport private-vlan host-association 100 102

no shutdown

exit

end

Router:

enable

configure terminal

interface fa0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

end

Explanation of the Diagram

- PC1 and PC4 (Isolated)

  - Cannot talk to PC2 or PC3.

  - Can only communicate with the router/server (promiscuous port).

- PC2 and PC3 (Community)

  - Can talk to each other.

  - Can talk to router/server (promiscuous port).

  - Cannot talk to PC1 or PC4 (isolated).

- Router/Server (Promiscuous Port)

  - Can communicate with all PCs.

✅ Simple takeaway:

- Promiscuous = gateway/server

- Isolated = fully private

- Community = shared group