61_Configure SPAN (Port Mirroring)

1️⃣ What is SPAN?

- SPAN (Switched Port Analyzer) is a feature used to copy traffic from one port or VLAN to another port.

- The copied traffic is sent to a monitoring device.

- Commonly used for packet analysis, troubleshooting, and security monitoring.

👉 In simple words:
SPAN = Traffic copy for monitoring

2️⃣ Why Do We Use SPAN?

We use SPAN when we want to:

- See live traffic passing through a switch port

- Troubleshoot network issues

- Capture packets using tools like Wireshark

- Detect security threats

- Analyze performance problems

Without SPAN, switch traffic stays inside the switch and cannot be captured.

3️⃣ Where is SPAN Used?

- Network troubleshooting

- Security monitoring

- IDS/IPS systems

- Packet capture (Wireshark)

- Data centers & enterprise networks

4️⃣ Types of SPAN

- Local SPAN

  - Source and destination ports are on the same switch

- Remote SPAN (RSPAN)

  - Traffic is sent to another switch using a SPAN VLAN

- Encapsulated RSPAN (ERSPAN)

  - Traffic is sent over IP network (advanced)

👉 For Packet Tracer, we use Local SPAN only.

5️⃣ Advantages of SPAN

- No need to physically break the network

- Real-time traffic monitoring

- Helps in fast troubleshooting

- Supports security tools

- Easy to configure

6️⃣ Disadvantages of SPAN

- Can increase CPU load on switch

- Not always 100% accurate under heavy traffic

- Destination port becomes monitor-only

- Misconfiguration can affect performance

7️⃣ Simple Example

- PC1 is sending traffic

- Switch copies PC1’s traffic

- PC2 (monitoring PC) receives a copy

- PC2 cannot send traffic, only listens

8️⃣ One-Line Purpose (Exam / Interview Ready)

SPAN is used to mirror switch traffic to a monitoring port for analysis and troubleshooting.

Configuration:

enable

configure terminal

monitor session 1 source interface fa0/1 both

monitor session 1 destination interface fa0/2

end

show monitor session 1

Remove Span:
show monitor session 1