62_Configure RSPAN (Remote SPAN)
1️⃣ What is RSPAN?

- RSPAN (Remote Switched Port Analyzer) is an extension of SPAN.

- It allows you to mirror traffic from one switch and send it to another switch for monitoring.

- The mirrored traffic travels through a special VLAN called RSPAN VLAN.

👉 Simple words:
RSPAN = SPAN, but monitoring device is on a DIFFERENT switch

2️⃣ Why Do We Use RSPAN?

We use RSPAN when:

- The monitoring device (Wireshark / IDS / Firewall) is not connected to the same switch as the source traffic.

- The network is large or distributed.

- Centralized monitoring is required.

👉 In real networks, you cannot always plug Wireshark into the same switch.

3️⃣ How RSPAN Works (Concept)

- Traffic is captured on Source Switch

- Traffic is placed into a special RSPAN VLAN

- That VLAN is trunked across switches

- Destination Switch receives the traffic

- Traffic is sent to monitoring port

4️⃣ RSPAN VLAN (Very Important)

- RSPAN uses a dedicated VLAN

- This VLAN must be:

  - Created on all switches

  - Configured with remote-span

- No normal user traffic should use this VLAN

Example:

vlan 999

remote-span

5️⃣ Why Not Use Normal VLAN for RSPAN?

- Normal VLANs forward traffic normally

- RSPAN VLAN:

  - Only carries mirrored traffic

  - Prevents loops

  - Prevents accidental access

6️⃣ Difference: SPAN vs RSPAN

| Feature | SPAN | RSPAN |
| --- | --- | --- |
| Monitoring switch | Same switch | Different switch |
| VLAN required | No | Yes (RSPAN VLAN) |
| Scalability | Limited | High |
| Usage | Small networks | Large networks |

7️⃣ Advantages of RSPAN

- Centralized monitoring

- Works across multiple switches

- No need to move monitoring device

- Useful for SOC / NOC teams

8️⃣ Disadvantages of RSPAN

- Extra VLAN overhead

- Uses trunk bandwidth

- Can affect performance if overused

- Requires careful VLAN configuration

9️⃣ Real-World Use Case (with SolarWinds)

- SolarWinds raises alert on Switch A

- Wireshark server is connected to Switch B

- Engineer enables RSPAN

- Traffic from Switch A reaches Wireshark on Switch B

- Root cause identified

🔟 One-Line Purpose (Exam / Interview Ready)

RSPAN is used to remotely mirror switch traffic to a monitoring device located on another switch using a special VLAN.

⚠️ VERY IMPORTANT (Like PVLAN & SPAN)

❌ RSPAN is NOT supported in Cisco Packet Tracer
✅ Supported on real Cisco switches / GNS3 / EVE-NG

Configuration:

RSPAN (Remote SPAN) – FULL CONFIGURATION

Topology Used

- Switch-A → Source switch (traffic to monitor)

- Switch-B → Destination switch (Wireshark / monitoring PC)

- RSPAN VLAN → 999

- Source Port → Fa0/1 on Switch-A

- Monitor Port → Fa0/2 on Switch-B

- Inter-switch link → Trunk

STEP 1️⃣ – Create RSPAN VLAN (ON BOTH SWITCHES)

enable

configure terminal

vlan 999

remote-span

end

STEP 2️⃣ – Configure Trunk Between Switches (BOTH SIDES)

enable

configure terminal

interface fa0/24

switchport mode trunk

switchport trunk allowed vlan add 999

no shutdown

end

STEP 3️⃣ – RSPAN SOURCE CONFIG (Switch-A)

enable

configure terminal

monitor session 1 source interface fa0/1 both

monitor session 1 destination remote vlan 999

end

STEP 4️⃣ – RSPAN DESTINATION CONFIG (Switch-B)

enable

configure terminal

monitor session 1 source remote vlan 999

monitor session 1 destination interface fa0/2

end

STEP 5️⃣ – VERIFY

show monitor session 1

REMOVE RSPAN

enable

configure terminal

no monitor session 1

end