63_Configure VLAN ACLs (VACL)

1️⃣ What is VACL?

- VACL (VLAN Access Control List) filters traffic inside a VLAN

- Applied to a VLAN, not to an interface

- Controls host-to-host traffic in the same VLAN

👉 Simple words:
VACL = firewall rules for devices inside the same VLAN

2️⃣ Why Do We Need VACL?

Normally:

- Devices in same VLAN can talk freely

- Router / Firewall is not involved

Problem:

- One infected PC can attack others

- No security inside VLAN

👉 VACL adds security inside the VLAN

3️⃣ What Problem Does VACL Solve?

| Without VACL | With VACL |
| --- | --- |
| Same VLAN = full access | Same VLAN = controlled |
| No inspection | Traffic filtered |
| Security risk | Improved security |

4️⃣ Where is VACL Used?

- Data centers

- Server VLAN protection

- PCI-DSS environments

- East-West traffic control

5️⃣ VACL vs Port ACL vs Router ACL

| Feature | Router ACL | Port ACL | VACL |
| --- | --- | --- | --- |
| Applied on | Interface | Interface | VLAN |
| Same VLAN traffic | ❌ No | ❌ No | ✅ Yes |
| Hardware based | ❌ | ❌ | ✅ |

6️⃣ How VACL Works (Simple Flow)

- Traffic enters VLAN

- VACL checks rules

- Permit or deny

- Action taken inside VLAN

7️⃣ Advantages

- Filters same-VLAN traffic

- No routing needed

- High performance (hardware-based)

- Granular security

8️⃣ Disadvantages

- Complex configuration

- Hard to troubleshoot

- Platform dependent

- Not supported in Packet Tracer

9️⃣ Real-World Example

- VLAN 10 → Servers

- Allow only App Server → DB Server

- Block all other hosts in VLAN 10

🔟 One-Line Purpose

VACL is used to control and secure traffic between devices within the same VLAN.

⚠️ IMPORTANT

❌ NOT supported in Cisco Packet Tracer
✅ Supported on real Cisco switches / GNS3 / EVE-NG

VACL Configuration

Scenario

- VLAN 10

- PC1 → 10.10.10.10

- PC2 → 10.10.10.20

- Block PC1 → PC2

- Allow all other traffic

STEP 1️⃣ – Create VLAN

enable

configure terminal

vlan 10

name USERS

end

STEP 2️⃣ – Assign Ports to VLAN

enable

configure terminal

interface fa0/1

switchport mode access

switchport access vlan 10

no shutdown

interface fa0/2

switchport mode access

switchport access vlan 10

no shutdown

end

STEP 3️⃣ – Create Extended ACL (MATCH TRAFFIC)

enable

configure terminal

ip access-list extended BLOCK_PC1_PC2

deny ip host 10.10.10.10 host 10.10.10.20

permit ip any any

end

STEP 4️⃣ – Create VLAN ACCESS MAP (VACL)

enable

configure terminal

vlan access-map VACL_USERS 10

match ip address BLOCK_PC1_PC2

action drop

vlan access-map VACL_USERS 20

action forward

end

STEP 5️⃣ – APPLY VACL TO VLAN

enable

configure terminal

vlan filter VACL_USERS vlan-list 10

end

STEP 6️⃣ – VERIFY

show vlan access-map

show vlan filter

REMOVE VACL

enable

configure terminal

no vlan filter VACL_USERS vlan-list 10

no vlan access-map VACL_USERS

no ip access-list extended BLOCK_PC1_PC2

end

IMPORTANT NOTES

- VACL works inside VLAN

- Router is NOT involved

- Deny = drop silently

- Permit = forward normally

INTERVIEW ONE-LINE

VACL filters traffic between hosts in the same VLAN, providing internal VLAN security.