64_Configure QoS on switch ports

1️⃣ What is QoS?

- QoS = Quality of Service

- A mechanism to prioritize certain types of traffic on a network.

- Ensures important traffic gets bandwidth and low latency, while less critical traffic may be delayed.

👉 Simple words:
QoS = Traffic management / traffic priority on switch ports

2️⃣ Why Do We Use QoS?

We use QoS to:

- Prioritize voice and video traffic over normal data

- Prevent network congestion

- Ensure critical applications get required bandwidth

- Improve user experience for delay-sensitive apps

3️⃣ Common Use Cases

- VoIP phones → priority over web browsing

- Video conferencing

- Critical business applications → ERP, DB access

- Bandwidth-limited links → MPLS, WAN links

4️⃣ QoS Mechanisms

- Classification → Identify traffic type (IP, VLAN, protocol)

- Marking → Assign priority (DSCP, CoS)

- Policing / Shaping → Control rate of traffic

- Scheduling → Decide which traffic goes first (Priority Queues, Weighted Round Robin)

5️⃣ QoS on Switch Ports

- Switch port can prioritize traffic using CoS (Layer 2) or DSCP (Layer 3)

- Modes:

  - Access port → assign CoS value

  - Trunk port → preserve CoS for VLAN traffic

- Common commands: mls qos, priority-queue out

6️⃣ Advantages

- Ensures voice/video quality

- Avoids packet loss for critical traffic

- Efficient bandwidth utilization

- Reduces jitter and latency

7️⃣ Disadvantages

- Configuration complexity

- Misconfiguration can block important traffic

- Not all switches support full QoS features

- Needs ongoing monitoring

8️⃣ Real-World Example

- VoIP phone on Fa0/1 → priority 5 (CoS)

- PC on Fa0/2 → default priority

- During congestion, voice traffic is sent first

9️⃣ One-Line Purpose

QoS ensures that critical traffic receives priority and sufficient bandwidth on switch ports.

10️⃣ Devices Required

- 1 Switch

- 2 PCs

- 1 VoIP Phone (optional for simulation in labs)

Configuration:

STEP 1️⃣ – Enable QoS on the Switch

enable

configure terminal

mls qos

end

STEP 2️⃣ – Configure Access Port for VoIP Phone

enable

configure terminal

interface fa0/1

switchport mode access

switchport access vlan 10

mls qos trust cos

priority-queue out

end

Explanation:

- mls qos trust cos → Trust CoS value from VoIP device

- priority-queue out → Enable priority queue on egress for voice

STEP 3️⃣ – Configure Access Port for PC (Default)

enable

configure terminal

interface fa0/2

switchport mode access

switchport access vlan 10

mls qos trust cos

end

Note:

- PC traffic uses default CoS (no high priority)

STEP 4️⃣ – Configure Trunk Port (If Connecting to Another Switch or Router)

enable

configure terminal

interface fa0/24

switchport mode trunk

switchport trunk allowed vlan 10

mls qos trust cos

priority-queue out

end

Explanation:

- Preserve CoS values on VLAN traffic

- Enable priority queue for important traffic

STEP 5️⃣ – Verify QoS Configuration

show mls qos

show mls qos interface fa0/1 statistics

show mls qos maps

STEP 6️⃣ – Optional: Remove QoS

enable

configure terminal

no mls qos

end

INTERVIEW / EXAM READY LINE

QoS on switch ports prioritizes critical traffic like voice/video and ensures bandwidth and low latency.